Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRIC CIRCUITS AND NETWORKS				      Time : 3 hours

Subject Code:	10EE201										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	If a 200 Ω resistor is connected across the terminals of a 5 V battery what is the power dissipated in the resistor?

2.	The number of independent loops for a network with ‘n’ nodes and ‘b’ branches is _________.

3.	Superposition theorem is valid only for __________.

4.	What is meant by duals and duality?

5.	What is the phase angle of a series RLC circuit at resonance?

6.	If the lower cut-off frequency is 2200 Hz and the upper cut-off frequency is 2400 Hz, what is the bandwidth?

7.	Write the current relation in star and delta network.

8.	Mutually induced voltage is present independently of, and in addition to, the voltage due to self induction. Say true or false.

9.	Which parameters are widely used in transmission line theory?

10.	A band pass filter may be obtained by using a high pass filter followed by a low pass filter. Say true or false?

PART – B  (5 x 3 = 15 MARKS)

11.	An electric heater draws 1500 W from a 230 V supply. What power will it take if the voltage is 220 V? What is the resistance of the heater?

12.	State maximum power transfer theorem.

13.	List the restrictions on pole zero locations for driving point functions.

14.	Define coefficient of coupling.

15.	Draw and write the overall ABCD parameters for a two port networks in cascade.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the current delivered by the source in the circuits shown in the following two 	figures.

[P.T.O.]

(OR)

17.	With a suitable circuit explain nodal analysis method.

18.	How to verify the reciprocity theorem? Explain.

(OR)

19.	Using Thevenin’s theorem how to find unknown current through any one resistor? Describe in 	detail.

20.	Derive the sinusoidal response of a RLC series circuit.

(OR)

21.	A 50µF capacitor and 20000 ohm resistor are connected in series across a 100 V battery at         t = 0. At t = 0.5 s, the battery voltage is suddenly increased to 150 V. Find the charge on capacitor at t = 0.75 s.

22.	Derive the coefficient of coupling of a coupled circuit.

(OR)

23.	A three-phase delta connected RYB system with an effective voltage of 400 V, has a balanced load with impedances 3+j4Ω. Calculate the:

a.	phase currents  	b.	line currents 		c.	power in each phase. 				   (5+5+5)

24.	The Z parameters of a two-port network are Z11=10Ω, Z22=15Ω; Z12=Z21=5Ω. Find the equivalent T network and ABCD parameters.

(OR)

25.	Explain low pass and high pass filters with suitable diagrams.