Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title:	DIGITAL SIGNAL PROCESSING						       Time: 3 hours

Subject Code:	10EE202	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is the overall impulse response h(n)when two systems with impulse responses h1(n) and h2(n) are connected in parallel?

2.	What is meant by step response of a system?

3.	What is the main advantage of FFT over direct computation of DFT?

4.	State the periodicity property of DFT.

5.	A causal and stable IIR filter ________ (can/cannot) have linear phase.

6.	Poles of Chebyshev filter lie on an ________(Circle/Ellipse)

7.	What is the necessary and sufficient condition for linear phase characteristic in FIR filter.

8.	What is the reason that FIR filter is always stable?

9.	Write two applications of signal processors.

10.	What are the different stages of pipelining in a processor?

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between passive and lossless systems.

12.	The first five points of the 8-point DFT of a real valued sequence are 	28,−4+j9.5,−4+j4,−4+j1.56,−4. Determine the remaining three points.

13.	Realize y(n)+y(n−1)+14y(n−2)=x(n)  in cascade form network.

14.	What are the advantages and disadvantages of FIR filter?

15.	What are the advantages and disadvantages of VLIW architecture?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain the various types of classification of DT signals with suitable examples. 		(8)

b.	Sketch the following signals:

i)	x(n) = u(n) – u(n+2)		ii)	x(n) = (n)  (n+2)							(2+2)

c.	Determine the Energy of the signal x(n) = 2ej3n.		                               			(3)

(OR)

17.	a.	Check whether the system y(n) = cos[x(n)] is 									(10)

i.	Static or dynamic		ii.	Linear or non-linear		iii.	Causal or non-causal

iv.	Time Invariant or time variant		v.	Stable or unstable

b.	Determine the impulse response for the cascade of two LTI systems having impulse 	responses h1(n)=12nu(n)and h2(n)=14nu(n).								(5)

18.	Find the DFT of a sequence using DIT algorithm.

(OR)

19.	Determine the output response y(n) if h(n) = {1, 1, 1}and x(n) = {1,2,3,1}by using

a.	Linear convolution 		b.	Circular convolution

c.	Linear convolution using circular convolution.

[P.T.O]

20.	The specifications of the desired low pass IIR filter is  0.8≤|H(ejw)|≤1.0,0≤w≤0.2π

|H(ejw)|≤0.2, 0.6π≤w≤π.	Design a Butterworth digital filter using impulse invariant transformation.

(OR)

21.	a.	Apply the bilinear transformation for Ha(s)=2(s+1(s+2)with T = 1 sec. and find H(Z).

(3)

b.	For the analog transfer function Ha(s)=2(s+1)(s+2) determine H(Z) using impulse 	invariance method. Assume T = 1 sec.                  								(4)

c.	Give direct form – I and direct form – II structure of second order system realization.	(8)

22.	Design a filter with Hdejw=e−j3w,−π4≤w≤π4 =0,π4≤|w|≤π using a Hanning window with N = 7.

(OR)

23.	Explain how is design of linear phase FIR filter done by frequency sampling method?

24.	a.	Describe the features of Von Neumann architecture and Harvard architecture. 		(8)

b.	Explain the criterion on the selection of digital signal processors.                  			(7)

(OR)

25.	Write notes on the following:

a.	Hardware Multiplier – Accumulator.				   	       						(7)

b.	Special instructions for digital signal processors.				       					(8)