Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRONIC CIRCUITS								      Time : 3 hours

Subject Code:	10EI202											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	When a 50 Hz sinusoidal voltage is applied to the input of a fullwave rectifier, the output frequency is ______________.

2.	Dc restorer is the other name for____________.

3.	Reverse saturation current is due to _____________.

4.	hfe is given by _____________.

5.	Dc load line is a straight line drawn on the collector characteristic curves between ________ and _________.

6.	In Base bias Q varies with ____________.

7.	The constant current area of FET lies between ____________ and ___________.

8.	The JFET is  Bipolar/Unipolar.

9.	Oscillator operates in the principle of _______________ feedback.

10.	The number of RC pair used in RC phase shift oscillator is _____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define ripple voltage and ripple factor.

12.	Mention a few methods used for biasing.

13.	Draw the diagram for Voltage divider bias.

14.	What is pinch off voltage?

15.	Define Barkhausen criteria.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the expressions for rectification efficiency, TUF, and regulation of a fullwave rectifier.

(OR)

17. 	Explain the concept of Clipper in detail and discuss the same with the addition of bias voltage.

18.	a.	Draw the low frequency hybrid equivalent circuit for CE and CB amplifier.			    (5)

b.	Give the approximate h-parameter conversion formulae for CB and CC configuration in 			terms of CE.																	  (10)

(OR)

19.	Draw the circuit diagram of small signal CE amplifier circuit and give its equivalent hybrid model. What is the role of Cc and Ce?

20.	Draw the small-signal equivalent circuit for an emitter follower stage at low frequencies and obtain the voltage gain.

(OR)

21.	Explain Class A amplifier and explain its efficiency range with and without the use of transformer.

22.	Sketch the small-signal low-frequency circuit of a CS amplifier and derive the expression for the voltage gain.

[P.T.O.]

(OR)

23.	Explain different types of biasing used in FET.

24.	a.	Explain negative feedback with the help of the emitter follower as an example. Why is the 	emitter follower so called?														    (8)

b.	The gain of an amplifier is decreased to 10,000 with negative feedback from its gain of 	60,000. Calculate the feedback factor. Express the amount of negative feedback in dB.  (7)

(OR)

25.	Explain monostable and astable multivibrators in detail.