Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	SIGNALS AND SYSTEMS							      	 Time : 3 hours

Subject Code:	10EI204										       	  Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Find the odd and even components of sequence x(n)={1,1,0.5}

2.	Define a casual system.

3.	Find the stability of system represented by h(n)=(0.99)n u(n+2).

4.	What is the overall impulse response h (n), when two systems with impulse response h1(n) and h2(n) are connected in series and parallel?

5.	When a periodic signal is said to have half wave symmetry?

6.	If Fourier series coefficients of x(t) are cn ,then the Fourier series coefficients of x(t) is given by ________.

7.	The DFT of the sequence x(n)=(nn0) is

8.	According to Parseval’s theorem xn2 is

9.	The Z transform f(z) of the function f(nT)= anT

10.	The region of convergence of Z-transform of the sequence 56nun−65nu−n−1is

PART – B  (5 x 3 = 15 MARKS)

11.	Define and distinguish energy and power signal.

12.	Find the convolution using the matrix method  x1(n)={1, –2, 3, 1} and x2(n)={2, –3, –2}

13.	State the linearity and time reversal properties of continuous time Fourier series.

14.	Explain the circular convolution property of discrete time Fourier transform.

15.	Find the Laplace transform and ROC of the signal e−atut.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Sketch the following signals.                                                                                         	(8)

i.	–2u(t–1)					ii.	r(–t+2)

iii.	3r(t–1)					iv.	πt+3

b. 	Find whether the following signals are periodic or not      							(7)

i.	x(t)=2 cos(10t+1)sin(4t1)		ii.	cos60πt+sin50πt

iii.	3cos4t+2sin2πt

(OR)

17.	a. 	Check whether the system y(n)=cos[x(n)]                                                                     	(8)

i.	static or dynamic						ii.	linear or nonlinear

iii.	causal or non causal					iv.	time invariant or time variant

[P.T.O]

b. 	Find the Even and odd components of the following signals.                                 		(7)

i.	x(t) =cost +sint + cost sint				ii.	xn=−2, 1, 2, −1, 3↑

18.	a. 	Determine the output response y(n) using graphical method, if x(n)={1, 2, 3, 2};     				h(n)={1, 2, 2}.    																(9)

b. 	State and explain any three properties of convolution.                                           		(6)

(OR)

19.	a. 	Find the convolution of the following signals:                                                        		(8)			x(n)=cosπn u(n); hn=12nun.

b. 	Determine the response of the system with impulse response h(t)=u(t) for the input x(t)= 			u(t).                                                                                                             		(7)

20.	a. 	Find cosine Fourier series of the half wave rectified sine function.                       		(9)

b. 	State and prove Parseval’s theorem.                                                                			(6)

(OR)

21.	a. 	Find the trigonometric Fourier series for the periodic signal x(t).                          		(10)

b. 	Explain power representation using the Fourier series.                                  			(5)

22.	a. 	State any three properties of Discrete –time Fourier series.                                   		(6)

b.	Find DTFT of:                                                                                                         		(9)

i.	u(n–k)					ii.	an u(n)		iii.	(n+2) – (n–2)

(OR)

23.	a. 	State and explain sampling theorem, also explain aliasing effect.                         		(6)

b. 	Find the frequency response of I order system described by difference equation.

y (n)=ay(n–1)+x(n). Plot magnitude and phase response for a=0.5.                           	(9)

24.	a. 	Find the Laplace transform of the signal xt=e−3tut+e−2tutand find ROC. 		(7)

b. 	Find the Z transform of the signalxn=sinω0nunand find ROC.                   		(8)

(OR)

25.	a. 	Explain the convolution property of Z transform.                                                   		(5)

b. 	Find the inverse Z transform of the following                                                       		(10)

xz=z21−azz−a        ROC: a < |z| <1a .