Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	SIGNAL CONDITIONING CIRCUITS					      Time : 3 hours

Subject Code:	10EI206											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What are the different kinds of packages of IC741?

2. 	A circuit whose output is proportional to the difference between the input signals is considered to be which type of amplifier?

3. 	Mention some of the linear applications of op-amp.

4. 	An astable multivibrator is also known as a ____________.

5. 	Op-amps used as high- and low-pass filter circuits employ which configuration?

6. 	An instrumentation amplifier has a high

a) output impedance 	b) power gain    		c) CMRR    			d) supply voltage

7. 	What is (are) the principal area(s) of application for isolation amplifiers?

a) medical 			b) power plant 		c) automation 		d) all of the above

8. 	Give some examples of monolithic IC voltage regulators.

9. 	Which of the following applications include a phase-locked loop (PLL) circuit?

a) Modems 			b) AM decoders 		c) Tracking filters 	d) All of the above

10. 	Which of the following devices is (are) a component of a digital-to-analog converter?

a) Integrator   		b) Comparator		c) Digital counters 	d) All of the above

PART – B  (5 x 3 = 15 MARKS)

11. 	List out the ideal characteristics of Op-amp.

12. 	Draw the circuit of inverting summer with the output equation.

13. 	Draw and list the features of buffer amplifier.

14. 	Give the classification and advantages of voltage regulators.

15. 	The timing components for a PLL are 15 k and 220 pF. Calculate the free-running frequency.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the circuit diagrams and derive the gain of inverting and non-inverting operational amplifier

(OR)

17.	Explain and derive the condition for DC-characteristics of an operational amplifier.

18.	Draw the circuit diagram of op-amp differentiator, integrator and derive an expression for the  output in terms of the input.

(OR)

19.	Draw the block diagram of an Astable multivibrator using 555 timer and derive an expression for its frequency of oscillation.

20. 	Explain the working of an instrumentation amplifier with a circuit. Give its characteristics and applications.

(OR)

[P.T.O.]

21.	Draw the circuit of a first order and second order Butterworth active low pass filter and derive its transfer functions.

22.	Draw and explain the functional block diagram of a 723 regulator.

(OR)

23.	Explain the working of a four quadrant multiplier.

24.	Explain working of PLL using appropriate block diagram and explain any one application of the same.

(OR)

25.	With neat block diagram, explain the successive approximation A/D converter.