Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DIGITAL CONTROL SYSTEMS						      Time : 3 hours

Subject Code:	10EI213											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the fastest ADC.

2.	Write the need for ZOH device.

3.	What is the Z transform for unit step input?

4.	Write the modified Z transform for e−at.

5.	Write the condition for observability.

6.	Write the control law for state feedback.

7.	What do you mean by quantization?

8.	State the expression for bilinear transformation.

9.	Write any one digital form of PID algorithm.

10.	Write the expression for desired transfer function of Dead beat algorithm.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define Shannon’s sampling theorem.

12. 	What is Inverse Z transform?

13. 	State the conditions for controllability of a system.

14. 	State the necessary condition for Jury’s stability criteria.

15. 	What is ringing?

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Sketch the block diagram of sampled data control system and write the functions 	performed by every block of the same.                      								    (8)

b.	Explain the factors involved in the selection of A/D, D/A convertors.         			    (7)

(OR)

17. 	Explain the functioning of any one ADC and DAC.

18. 	a.	Write the need and significance of modified Z transform with examples. 			    (8)

b.	Obtain the modified Z transform of

Gps=e−0.76s0.4s+1.		                                                                             			    (7)

(OR)

19. 	Obtain the step response of sampled data system with ZOH.

Gps=20.1s+1s+1.

20. 	Elaborate the significance of controllability and observability and test the same for the following system.

A=−2−311	B=10		C=01

[P.T.O.]

(OR)

21. 	Place the closed loop poles of the system with

A=1111	B=10

at 0 and 0.5 using a state feedback controller.

22.  	Determine the stability of the sampled data control system whose open loop transfer function is given by

AZ=Z2.45Z+12.45Z−1

(OR)

23. 	Describe the Quantization phenomena and discuss the effects, error caused in Quantization.

24. 	Describe the hardware, flowchart for implementation of control algorithms using microcontroller.

(OR)

25.	Derive the control law for a process with sampled data control system having a transfer function     Gps=1s+1for a unit step change in set point with deadbeat controller with

T = 0.5 Sec.