Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MECHANICS OF MACHINES – II					      	     Time : 3 hours

Subject Code:	10ME201								 		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is free body diagram of a mechanism?

2. 	Why the cranks of locomotive are generally at right angles to one another?

3.  	What is turning-moment diagram?

4. 	The ratio of the fluctuation of energy to the ___________ is called as coefficient of fluctuation of energy.

5. 	State the necessary conditions to achieve dynamic balancing.

6. 	What is the effect of swaying couple?

7. 	In a spring mass system, if the mass is halved and the spring stiffness is doubled, the natural frequency is ___________.

8. 	Why does the amplitude of free vibration gradually diminish in practical system?

9.  	At a nodal point in a shaft, the amplitude of torsional vibration is ___________.

10. 	State the conditions for torsionally equivalent system.

PART – B  (5 x 3 = 15 MARKS)

11. 	What are conditions for a body to be in equilibrium under the action of three forces and a torque with two forces?

12. 	A flywheel fitted to a steam engine has a mass of 800 kg. Its radius of gyration is 360 mm. The starting torque of the engine is 580 N-m and may be assumed constant. Find the kinetic energy of the flywheel after 12 seconds.

13. 	What are radial engines? How are the forces analyzed in radial engines?

14. 	Derive the natural frequency of the spring-mass system with inertia of the spring.

15. 	What are the constraints used in Holzer method to determine the natural frequency of the system?

PART – C  (5 x 15 = 75 MARKS)

16. 	In a four link mechanism shown in the figure, torque T3 and T4 have magnitudes of 30 N-m and        20 N-m respectively. The link lengths are AD = 800 mm, AB = 300 mm, BC = 700 mm and CD = 400 mm. For the  static equilibrium of the mechanism, determine the required input torque T2.

[P.T.O.]

(OR)

17. 	A single cylinder vertical engine has a bore of 300 mm and a stroke of 400 mm. The connecting rod is 1000 mm long. The mass of the reciprocating parts is 140 kg. On the expansion stroke with the crank at 30 from top dead centre, the gas pressure is 0.7 MPa. If the engine runs at 250 rpm, determine:

a.	Net force acting on the piston.

b.	Resultant load on the gudgeon pin.

c.	Thrust on the cylinder walls.

d.	The speed above which, other things remaining same, the gudgeon pin loads would be 	reversed in direction.

18. 	The torque delivered by a two stroke engine is represented by T = (1200 + 1400 sin  + 210 sin2 +21 sin3) N-m where  is the angle turned by the crank from the inner dead centre. The engine speed is 210 rpm. Determine the power of the engine and the minimum mass of the flywheel if its radius of gyration is 800 mm and the maximum speed is to be 1.5% of the mean.

(OR)

19. 	A machine punching 38 mm holes in 32 mm thick plate requires 7 N-m of energy per sq. mm of sheared area, and punches one hole in every 10 seconds. Calculate the power of the motor required. The mean speed of the flywheel is 25 meters per second. The punch has a stroke of 100 mm. Find the mass of the flywheel required, if the fluctuation of speed is not to exceed 3 % of the mean speed. Assume that the motor supplies energy to the machine at uniform rate.

20. 	Four masses A, B, C and D revolve at equal radial and are equally spaced along a shaft. The mass B is 7 Kg and the radial of C and D make angles of 90 and 240 respectively with the radius of B. Find the magnitude of the masses A, C and D and the angular position of A so that the system may be completely balanced.

(OR)

21. 	A four crank engine has two outer cranks set at 120 to each other, and their reciprocating masses are each 400 kg. The distance between the planes of rotation of adjacent cranks are 450 mm, 750 mm and 600 mm. If the engine is to be in complete primary balance, find the reciprocating mass and the relative angular position for each of the inner cranks. If the length of each crank is 300 mm, length of each connecting rod is 1.2 m and the speed of rotation is 240 rpm, what is the maximum secondary unbalanced force?

22. 	A disc of a torsional pendulum has a moment of inertia of 600 Kg – cm2 and is immersed in a viscous fluid. The brass shaft attached to it is of 10 cm diameter and 40 cm long. When the pendulum is vibrating, the observed amplitudes on the same side of the rest position for successive cycles are 9, 6 and 4. Determine:

a.	Logarithmic decrement.

b.	Damping torque at unit velocity.

c.	Periodic time of vibration.

Take G = 4.4 x 1010 N / m2.

(OR)

23. 	A heavy machine, weighing 3000 N, is supported on a resilient foundation. The static deflection of the foundation due to weight of the machine is found to be 7.5 cm. It is observed that the machine vibrates with amplitude of 1 cm when the base of the foundation is subjected to harmonic oscillation at the un-damped natural frequency of the system with amplitude of 0.25 cm. Find:

a.	damping constant of the foundation.

b.	dynamic force amplitude on the base.

c.	amplitude of the displacement of the machine relative to the base.

24. 	A steel shaft ABCD 1.5 m long has flywheel at its ends A and D. The mass of the flywheel A is 600 kg and has a radius of gyration of 0.6 m. The mass of the flywheel D is 800 kg and has a radius of gyration of 0.9m. The connecting shaft has diameter of 50 mm for the portion AB which is 0.4m long; and has a diameter of 60 mm for the portion BC which is 0.5 m long; and has a diameter ‘d’ mm for the portion CD which is 0.6 m long. Determine:

a.	the diameter ‘d’ of the portion CD so that the node of the torsional vibration of the system will be at the centre of the length BC.

b.	the natural frequency of the torsional vibrations

Take modulus of rigidity for the shaft material is 80 GN/m2.

(OR)

25. 	An IC engine drives a centrifugal compressor through a pair of pinion and gear. The compressor speed is one-fourth of the engine. The steel shaft from the flywheel of engine to the pinion is 50 mm in diameter and 0.4 m in length. The shaft from the gear to compressor is 60 mm in diameter and 0.5 m in length. If the mass moment of inertia of flywheel, pinion, gear and impellor are 500 kg-m2, 5 kg-m2, 15 kg-m2 and 500 kg-m2 respectively, determine the natural frequency of torsional vibration. Take G = 0.8 x 1011 N/m2.