Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	INSTRUMENTATION & CONTROL SYSTEMS			      Time : 3 hours

Subject Code:	10ME202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Unit used to measure nuclear diameter is

a.	Pico meter 		b.	Fermi		c.	Micron 		d.	millimeter

2.	Which of the following is not a physical quantity?

a.	Kelvin			b.	Candela		c.	Henry		d.	all the above

3.	Which of the following statements is correct?

a.	Sensors and actuators are both examples of transducers.

b.	Sensors and transducers are both examples of actuators.

c.	Actuators and transducers are both examples of sensors.

d.	none of these

4.	What term describes the maximum expected error associated with a measurement or a sensor?

a.	Resolution.		b.	Range.		c.	Accuracy.	d.	Precision.

5.	For measuring current at high frequency we should use

a.	moving iron instrument		b.	thermocouple instrument

c.	electrostatic instrument		d.	none of these

6.	Which of the following processes are thermodynamically reversible?

a.	none						b.	throttling free expansion

c.	isothermal and adiabatic		d.	constant volume and constant pressure

7.	Which of the following instruments is used for measurement of torque?

a.	manometer 					b.	dynometer

c.	flowmeter					d.	all of these

8.	Train gauge is insensitive to

a.	tensile force					b.	comprehensive force

c.	lateral force 					d.	none of these

9.	The term control system refers to

a. 	any system with high gain

b.	system having a controlled input only

c.	system with a provision for controlling the output

d.	none of these

10.	In control system, servomechanism is used for system dealing with

a.	position

b.	velocity

c.	acceleration

d.	any of these

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain about the input quantities.

12. 	Explain about pyrometers.

13. 	Discuss about the Mc Leod gauge.

14. 	Explain about measurement of PH-PH meters.

15. 	Differentaite between open loop and closed loop system.

[P.T.O.]

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain in detail about the methods of measurements.

(OR)

17. 	Define errors and explain in detail about the classification of errors.

18. 	What is a transducer? Explain any two types in detail.

(OR)

19. 	Explain in detail about the laws of thermocouple with a diagram.

20. 	Discuss in detail about classification of flow meters with diagram.

(OR)

21. 	Explain in detail about bourdon tube pressure gauge with diagram.

22. 	Describe in detail about Geiger Muller counter with diagram.

(OR)

23. 	What is a strain gauge? Explain in detail with diagram.

24. 	Discuss the different blocks of control system with diagram.

(OR)

25. 	Explain in detail about Routh and Hourwitz stability criteria.