Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	10MT203										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define signal.

2.	What is the basic difference between continuous-time signal and discrete-time signal?

3.	What is the difference between DFT and DTFT.

4.	Name any two properties of Fourier Transform.

5.	Expand the terms IIR and FIR with respect to systems.

6.	For a stable analog filter poles should lie on the __________half of s-plane.

7.	Mention three windowing techniques used in FIR filter design.

8.	What is the width of the mainlobe in rectangular window?

9.	What are the two types of quantization employed in a digital system?

10.	The two major methods of binary numbers are __________ and __________ representation.

PART – B  (5 x 3 = 15 MARKS)

11.	Test for the periodicity of  xn=e7πn.

12.	State and prove the Time Shifting property of DTFT.

13.	What are the requirements for an analog filter to be stable and causal?

14.	What is Gibb’s phenomenon?

15.	List some of word length effects in digital filters.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Differentiate energy and power signal.											    (6)

b.	Explain the following with respect to continuous-time LTI system:		i.	Stability

ii.	Causality 	iii.	Memory system.											    (9)

(OR)

17.	An LTI system has an impulse response h(n) = u(n+1)–u(n–10). Determine the output of the system when the input x(n) = – u(n)+2u(n–3) –u(n–6). Show the steps clearly with graphical representation.

18.	a.	Compute the Fourier Series coefficients for the following signal x(t) depicted in figure.

(10)

[P.T.O.]

b.	What are the advantageous features of FFT algorithm over DFT method?       		    (5)

(OR)

19.	Derive the FFT algorithm using decimation in time. Determine the DFT of the given data sequence  x(n) = { –1, 2, –3, 4, 9, –20, 12, 6} by decimation - in - time  FFT.

20.	a.	Distinguish between FIR and IIR filters.											    (6)

b.	Apply the bilinear transformation to Ha(s) = 2(s+1)(s+2) with T = 1 sec and find H(z).																		    				    (9)

(OR)

21.	a.	Explain briefly how the analog frequency is mapped to digital frequency in bilinear 	transformation related with IIR filter design.										    (8)

b.	What are advantages and disadvantages of bilinear transformation?					    (7)

22.	Design an ideal low pass FIR (linear phase) filter using rectangular window and Hamming window of N = 9 whose desired frequency response is

Hd (ω) = 1, −π3 ≤ ω ≤ π3

(OR)

23.	Draw the generalized structure of direct form - I and direct form - II realization of IIR filter. Obtain the direct form - I and direct form - II realization for the following system.

y(n)=34y(n−1)−18y(n−2)+x(n)+13x(n−1)

24.	a.	What is meant by finite word length effects in digital filters? 			     			    (7)

b.	Compare the fixed point and floating point number representations.                        	    (8)

(OR)

25.	a.	Explain briefly the limit cycle oscillation with respect to finite word length effect. 	    (8)

b.	Why rounding is preferred for quantizing the product?			 	      			    (7)