Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	AIRCRAFT STRUCTURES								      Time : 3 hours

Subject Code:	11AE202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	If n=2j–3 then the frame is called as ___________.

2. 	Fixed beam is a statically determinate beam. (True / False).

3. 	If the far end of a beam is fixed supported then the carry over factor is __________.

4. 	Give the expression for the stress due to gradually applied and suddenly applied load.

5. 	Give Euler’s critical formula.

6. 	Which failure theory is suitable for ductile material?

7. 	Shear center and centroid of a section coincides if ____________.

8. 	Shear force gradient is otherwise called as ___________.

9. 	What is a Wagner beam?

10. 	List out any two functions of ribs.

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate between statically determinate and indeterminate structure.

12. 	Write down the three moment equation in general form.

13. 	State Maxwell's Reciprocal theorem.

14. 	Define shear center and elastic axis.

15. 	Write short notes on shear resistant web beams.

PART – C  (5 x 15 = 75 MARKS)

16. 	Determine the forces in the members of the truss shown in the figure.

(OR)

17. 	Analyse the continuous beam shown in figure by the method of moment distribution.

[P.T.O.]

18. 	Analyze the continuous beam shown in the figure. Use strain energy method.

(OR)

19. 	State and prove Castigliano’s theorem.

20. 	Mention the applicability of Euler’s theorem. Derive the expression for Euler’s critical load, when the both the ends are pinned.

(OR)

21. 	A hollow mild steel  shaft having 100 mm external diameter and 50 mm internal diameter is subjected to a twisting moment of 8 KNm and bending moment of 2.5 KNm. Calculate the principal stresses and find direct stress, which acting alone, would produce the same.

a.	maximum elastic strain energy.

b.	maximum elastic shear strain energy, as that produced by the principal stresses acting 	together. Take poission’s ratio = 0.25.

22. 	Calculate the shear flow and shear center for the given section shown in fig.4.

(OR)

23. 	Find the shear flow and twist per unit length for the given section shown in the figure and given that  Torque =  75000 N-cm. The material used is Aluminium with Young’s modulus E = 70 Gpa and Poisson ratio = 0.3. Both the cells are Square in cross section.

24. 	a.	What are the various structural components contained in an aircraft? What are their 				functions? 																	    (8)

b.	Explain the construction of shear force and bending moment diagrams for a cantilever type 		of aircraft wing.																    (7)

(OR)

25. 	a.	Explain the pure and semi tension field beams.					 				    (8)	 b.	Differentiate between the monocoque and semi monocoque type fuselage construction.  (7)