Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	FLIGHT MECHANICS AND PERFORMANCE			      Time : 3 hours

Subject Code:	11AE204										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define geometric altitude.

2. 	If an aircraft senses the actual outside air pressure which corresponds to some standard altitude, then that altitude is called __________.

3. 	___________ is defined as the reciprocal of  bulk modulus of fluids.

4.	Minimum drag occurs when L/D is minimum. (True / False)

5. 	Define service ceiling.

6. 	What are the two methods for predicting fuselage contribution to longitudinal stability of airplane?

7. 	The magnitude of pitching moment coefficient, Cm, obtained per degree deflection of the elevator is termed as ___________.

8. 	The forward c.g location is limited by the influence of the ground on the downwash during _____________. (Landing or take off). Choose the correct answer.

9.	Define stick- fixed maneuvering point.

10. 	_________ control will be expressed as the change in rolling moment coefficient per degree deflection of the ailerons

PART – B  (5 x 3 = 15 MARKS)

11. 	Distinguish between Troposphere and Stratosphere.

12. 	What are the conditions for minimum drag and minimum power required for an airplane? Mention them in drag coefficients also.

13. 	What are the major contributions of the indirect effects of the running propellers on the static longitudinal stability?

14. 	What is the effect Aileron reversal?

15. 	What do you infer from auto rotation?

PART – C  (5 x 15 = 75 MARKS)

16. 	Briefly explain with the temperature-altitude plot, the structure of the standard atmosphere upto a 105 km altitude.

(OR)

17. 	What are the different types of drag acting on the flight vehicles and explain them.

18. 	An aircraft weighing 25 k N has a wing area 100m2 and its drag coefficient is CD= 0,016 + 0.04 CL2, calculate the minimum thrust required for straight and level flight, and the corresponding true air speed at sea level and at 10km (√σ=0.58). Calculate also the minimum power required and the corresponding true air speeds at the above conditions.

(OR)

19. 	Find out the velocity ratio (x) for max: R/C, when y1= (2TA/TL/Dmax) is 2 and 5 and also obtain the flight   velocity of air plane if its VL/Dmax= 600km/hr.

20. 	Derive the relation for Neutral point (N0) of an airplane at stick-fixed Power-off condition.

[P.T.O.]

(OR)

21. 	From fundamental, write down the equation for the Wing contribution to Static Longitudinal Stability (S.L.S) and from the simplified form explain how the location of c.g of airplane with respect to a.c of wing decides the S.L.S of the airplane.

22. 	Write short notes on the following:

a.	Adverse yaw effects. 															    (5)

b.	Aileron reversal. 																    (5)

c.	Weather cocking effect.														    (5)

(OR)

23. 	Discuss in detail the contribution of various components of the airplane on its static directional stability.

24. 	Write short notes on the following:

a.	Auto rotation. 																    (5)

b.	Dutch roll.	 																    (5)

c.	Spiral Divergence.															    (5)

(OR)

25. 	a.	Discuss various stability derivatives relevant to longitudinal dynamics.  			  (10)

b.	Discuss briefly about Phugoid motion. 											    (5)