Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	AERODYNAMICS										      Time : 3 hours

Subject Code:	11AE205										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List out the difference between velocity potential function and stream function?

2.	Determine the condition that the velocity components u= ax+dy and v= cx+dy will satisfy the equation of continuity.

3.	What is the practical importance of Kutta condition?

4.	Give the expression for the pressure coefficient; which holds for incompressible flow only.

5.	For the NACA 23012 airfoil, the design lift coefficient is ____________.

6.	Find Ф and ψ for the transformation w =Z2 .

7.	What phenomena cause induced drag?

8.	Define Critical Reynolds number.

9.	The point about which the wing pitching moment is independent of angle of attack is called________________.

10.	Define: Wash in and wash out.

PART – B  (5 x 3 = 15 MARKS)

11.	For the velocity field   u = y/(x2 + y2) and   v =  –x/(x2 + y2) Calculate the circulation 	round 	a circular path of radius 5m. Assume that u and v are in units of m/s.

12.	Define Magnus Effect and D’Alembert Paradox?

13.	Consider an NACA 2412 airfoil with a chord of 0.64 m in an air stream at standard sea level conditions. The free stream velocity is 70 m/s. The lift per unit span is 1254 N/m. Calculate the angle of attack and the drag per unit span.

14.	Briefly discuss on Boundary layer and boundary layer thickness

15.	What are the limitations of lifting line theory?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the continuity equation for unsteady three dimensional flows in partial differential form using a control volume approach.

(OR)

17.	a.	Define ‘stream function’ and ‘velocity potential function’. Derive expressions for stream 	functions of three elementary flows.											  (10)

b.	Consider an airfoil in a flow at standard sea level conditions with a free stream velocity of 	50 m/s. At a given point on the airfoil, the pressure is 0.9 x 105 N/m2. Calculate the 	velocity at this point. 											 				    (5)

18.	a.	Discuss in detail about the Real flow over smooth and rough cylinder with neat sketches.

(7)

b.	Consider the lifting flow over a circular cylinder with a diameter of 0.5 m. The free stream velocity is 25 m/s, and the maximum velocity on the surface of the cylinder is 75 m/s. The free stream conditions are those for a standard altitude of 3 km (ρ= 0.90926 kg/m3). Calculate the lift per unit span on the cylinder.									    (8)

[P.T.O.]

(OR)

19.	State Kutta – Joukowski theorem. Prove the theorem with respect to a lifting flow 	over 	a 	cylinder with neat sketches.

20.	a.	Explain how the Joukowski transformation is used to obtain a circular arc aerofoil and also 	obtain an expression for its radius. 												  (10)

b.	What is Karman – Treffetz airfoil? Explain with neat sketches.						    (5)

(OR)

21.	Derive the fundamental equation of thin airfoil theory. For flows over a flat plate show that the lift co-efficient is linearly proportional to angle of attack.

22.	a.	Obtain an expression for displacement thickness and momentum thickness of flow over a 			flat plate.																	  (10)

b.	Explain the Boundary Layer growth over a Flat plate. 					  		    (5)

(OR)

23.	Derive the Boundary layer equations for a steady, two dimensional incompressible flows.

24.	a.	State Biot Savart law and derive an expression for the velocity induced by an infinite 			vortex filament.																  (10)

b.	Consider a finite wing with an aspect ratio of 8 and a taper ratio of 0.8. The airfoil section is thin and symmetric. Calculate the lift and induced drag coefficients for the wing when it is at an angle of attack of 5°. Assume that δ = τ.							  		    (5)

(OR)

25.	Derive the fundamental equation of Prandtl’s lifting line theory and obtain an expression for induced drag for elliptic lift distribution.