Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DIGITAL ELECTRONICS								      Time : 3 hours

Subject Code:	11EC204										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State and prove De Morgan’s theorem.

2.	Simplify the following Boolean function to a minimum of literals: (X+Y) (X+Y).

3.	Mention the difference between DMUX and MUX.

4.	Define a combinational logic circuit.

5.	Give the excitation table of SR flip flop.

6.	Convert JK flip flop to D flip flop.

7.	Explain about state reduction.

8.	What is race around condition in latches?

9.	Distinguish PAL and PLA.

10.	What is meant by tristate capability?

PART – B  (5 x 3 = 15 MARKS)

11.	Express the function f(x, y, z)= xy+ xz in POS form.

12.	Draw a 4*16 decoder with two 3*8 decoders.

13.	Name the different types of triggering used in flip flop.

14.	What is the minimum number of flip flops required to implement a modulo 21 synchronous counter?

15.	Classify the logic families.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the prime implicants of the following function and verify using K map.

F= (A, B, C, D)= Σ (3, 4, 5, 7, 9, 13, 14, 15)

(OR)

17.	a.	Minimize the following using Quine Mclunskey method							(10)

Σm (0, 1, 2, 8, 9, 15, 17, 21, 24, 25, 27, 31)

b.	State and prove the associative law of Boolean algebra.							 (5)

18. 	Design the following circuits: Full Adder and Gray Code Converter.	      			  (10+5)

(OR)

19.	Implement the following multiple output combinational logic circuit using a 4-line to 16-line decoder.

f1 = Σ (1, 2, 4, 7, 8,11,12,13)

f2 = Σ (2, 3, 9, 11)

f3 = Σ (10, 1, 213, 14)

f4 = Σ (2, 4, 81)

20.	a.	Realize a JK flip flop using only NOR gates.										    (9)

b.	Explain the operation of SR flip flop with necessary diagrams.						    (6)

(OR)

[P.T.O.]

21.	a.	Explain the various steps involved in the analysis of synchronous sequential circuits with 	suitable example.																    (9)

b.	Compare Moore and Mealy circuits.											    (6)

22.	Design a MOD10 synchronous counter using JK flip flops. Write execution table and state table.

(OR)

23.	Design a synchronous counter with states 0, 1, 2, 3, 0.1…… using JK flip flops.

24.	Describe the concept, working and applications of PLD, PLA, Schottky TTL. 		   (5+5+5)

(OR)

25.	Explain the working of 3-input TTL totem pole NAND gate.