Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	COMMUNICATION THEORY AND SYSTEMS			      Time : 3 hours

Subject Code:	11EC206										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Amplitude modulation by square waves or rectangular binary pulses is referred to as

___________.

2. 	Double-sideband suppressed carrier signals are generated by a circuit called a __________.

3.	An AM demodulator can be implemented with a linear multiplier followed by a ___________ filter.

4. 	Which block of an AM receiver is used to produce an intermediate frequency for amplification and detection?

5. 	The shared block of a receiver that consists of AM and FM signals is the:

a.	mixer 		b.	IF amplifier 		c.	audio amplifier 		d.	discriminator

6. 	___________ is the most widely used FM demodulator.

7. 	A limiter is used in FM broadcast receiver in order to ___________.

8.	FM transmitters handle audio frequencies up to ___________ KHz.

9. 	Which type of multiple access method preferred in GSM Cellular systems?

10. 	Which of the following gives maximum probability error?

PART – B  (5 x 3 = 15 MARKS)

11. 	How many AM broadcast stations can be accommodated in a 100 kHz bandwidth
if the highest frequency modulating a carrier is 5 kHz?

12. 	Draw the block diagram of a method for generating a narrowband FM signal.

13. 	Define pre-emphasis and de-emphasis.

14. 	What are the characteristics of super heterodyne receivers?

15. 	State the applications of FDMA.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain Balanced modulator to generate DSB-SC signal.							  (10)

b.	Write short notes on Communication system block diagram. 						    (5)

(OR)

17.	a.	Draw the circuit diagram of Ring modulator and explain with its operation. 			  (10)

b.	Compare AM and FM.														    (5)

18.	Derive the expression for the frequency modulated signal. Explain what is meant by narrowband FM and wideband FM using the expression?

(OR)

19.	A 20 MHz is frequency modulated by a sinusoidal signal such that the maximum frequency deviation is 100 kHz. Determine the modulation index and approximate bandwidth of the FM signal for the following modulating signal frequencies,

a.	1 kHz 		b.	100 kHz 		c.	500 kHz.

[P.T.O.]

20.	a.	Explain Super heterodyne receiver with a neat diagram. 							  (10)

b.	Explain about the vharacteristics of Receivers. 									    (5)

(OR)

21.	a.	Write short notes on Tuned radio frequency receivers. 							    (8)

b.	Compare high level and low level transmitters. 									    (7)

22.	a.	Explain the significance of Pre-emphasis and De-emphasis in an FM system. 		    (7)

b.	Derive the output SNR for FM reception. 										    (8)

(OR)

23.	a.	Write short notes on thermal noise and short noise. 								    (8)

b.	Write notes on noise temperature and noise figure.								    (7)

24.	a.	Explain frequency translation and FDM. 										    (8)

b.	With a neat diagram explain about PSK. 										    (7)

(OR)

25.	How an FM can be obtained from PM and vice versa? Explain.