Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	LINEAR INTEGRATED CIRCUITS AND APPLICATIONS														      								      Time : 3 hours

Subject Code:	11EC208										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the properties of SiO2.

2.	What do you meant by MBE?

3.	What are the limitations of basic differentiator circuit?

4.	Name some applications of comparator.

5.	Give some examples of monolithic IC voltage regulators.

6.	What do you mean by Schmitt Trigger?

7.	Mention some commonly used active filters.

8.	List the applications of 555 timer in astable mode of operation.

9.	List out direct type ADC.

10.	Define capture range of PLL.

PART – B  (5 x 3 = 15 MARKS)

11.	What do you meant by MBE?

12.	What is the need for an instrumentation amplifier?

13.	Why current boosting is done in 723 regulators?

14.	List any four applications of 555 timer in astable mode of operation.

15.	Define conversion time of ADC.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about the processing steps involved in the fabrication of a capacitor.

(OR)

17.	Briefly explain the various types of IC packages. Write down the criteria for selecting IC packages.

18.	a.	Explain the application of op amp as integrator and differentiator.			 		  (10)

b.	What is sample and hold circuit? Why is it needed?					   			    (5)

(OR)

19.	a.	Discuss in detail about the operation instrumentation amplifiers.			 		  (12)

b.	List the applications of log amplifiers.											   (3)

20.	a.	Draw the functional block diagram of switching regulator and explain.				   (9)

b.	Design a step down switching regulator with the following specifications.			   (6)

Vin =12VDC

V0 = 5V at 500mA maximum

Vripple= 50mV or 1% of V0

(OR)

21.	a.	Design and explain triangular wave generator using Schmitt trigger and integrator circuit.																						    (9)

b.	Design Wien bridge oscillator for an output frequency of 965 Hz.					    (6)

[P.T.O.]

22.	Name the modes of operation of timer IC. Derive the expression of time delay for mono stable multivibrator.

(OR)

23.	Derive the frequency response of a first order Butterworth filter and plot its frequency response.

24.	a.	Derive the capture range for PLL where a simple RC network is used as a LPF.		  (10)

b.	Discuss about phase detector in brief.											    (5)

(OR)

25.	Discuss the operation of ladder network DAC with relevant circuit diagrams.