Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	PULSE AND WAVE SHAPING CIRCUITS				      Time : 3 hours

Subject Code:	11EC210										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The differentiator has __________ time constant.

2.	The clamper is also known as__________.

3.	A bistable multivibrator is a __________ circuit.

4. 	The other name of Schmitt trigger is __________.

5. 	If R1 = R2, β = 0.5, Vsat >> VD, then time period of pulse width of monostable  multivibrator is _____________.

6. 	The frequency deviation of VCO is directly proportional to __________.

7.	In __________ circuit, a pulse input is converted in to ramp.

8. 	If the restoration time of the sweep is zero, we get a __________.

9.	__________ feedback is employed in blocking oscillator.

10. 	The chopper is often called a __________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate between linear and non-linear wave-shaping circuits.

12. 	What is a Bistable circuit? What are the other names of a bistable multivibrator?

13. 	Why is monostable multivibrator also called a gating circuit?

14. 	What are the methods of generating a time-base waveform?

15. 	What are the advantages and disadvantages of the unidirectional diode gate?

PART – C  (5 x 15 = 75 MARKS)

16.	Classify different types of clipper circuits. Give their circuits and explain their operation with  the aid of transfer characteristics.

(OR)

17.	a.	Explain how a low pass circuit act as an integrator. 								  (8)

b.	Draw the high-pass circuit and explain its working. 								  (7)

18.	a.	With the help of a neat circuit diagram explain the working of an astable multivibrator.

(10)

b.	What is blocked condition in an astable multivibrator? How to overcome it?			  (5)

(OR)

19.	With the help of a neat circuit diagram, explain the working of an emitter coupled monostable multivibrator.

20.	With the help of neat circuit diagram and waveforms, explain the working of a Schmitt trigger. (OR)

21.	What do you mean by a regenerative comparator and non-regenerative comparator?  Explain with an example.

22.	Draw the circuit diagram of transistor Miller time base generator and explain its working.

(OR)

[P.T.O]

23.	With the help of a neat circuit diagram, explain the working of a simple current sweep.

24.	Draw the circuit diagram of a sampling gate with more than one control voltage and explain its working.

(OR)

25.	With the help of a circuit diagram and waveforms, explain Triggering Transistor blocking oscillators.