Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ANTENNAS AND WAVE PROPAGATION				      Time : 3 hours

Subject Code:	11EC211										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The vectors H or B may be derived from a suitable magnetic vector potential A through the relation ______________.

2.	Define Quarter Wave Monopole?

3.	What is the beam efficiency of a Dipole antenna?

4.	Loss aperture AL can be expressed as _____________.

5.	Draw the terminatedRhombicantenna.

6.	Front to Back ratio ofYagiuda antenna is _____________.

7.	Define sectoral E-plane horn and sectoral H-plane horn.

8.	What are all the various applications of corner reflector?

9.	Define Ground Wave?

10.	____________ Layer occurs during day light hours.

PART – B  (5 x 3 = 15 MARKS)

11.	Derive the “E” field expression for Quasi-Stationery field region.

12.	A thin dipole antenna is λ / 15 long. If it’s RL=1.5 ohm. Find Rr and its efficiency.

13.	Write notes on the uses of log periodic antenna.

14.	Write the differences between Slot antenna and its complementary Dipole antenna.

15.	What are all the possible modes of propagation? Give its principle.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the expression for power radiated and radiation resistance of current element.

(OR)

17.	a. 	Derive Eθ and Hφ field components from Δ x A = μ H for half wave dipole antenna.	(8)

b. 	A half wave dipole antenna is driven with a current of 0.5 A at the terminals calculate E 	and H, 1 km from the antenna at angles θ = 45 0 and 900.                        			     	(7)

18.	a. 	Calculate the gain of antenna with a circular aperture of diameter 3m at F=5 GHz.  	(5)

b. 	Derive the relationship between effective aperture, gain and directivity.				(10)

(OR)

19.	Show that the impedance value of folded dipole is n2 x 73 where ‘n’ is the number of half wave 	dipoles.

20.	Derive the voltage and current relation in parasitic antennas.

(OR)

21.	Derive α – the apex angle of log periodic antenna from log periodic dipole array.

22.	Derive the radiation intensities from an elemental area of a plane wave using Huygens            source.

(OR)

23.	Explain the various feed systems of reflector antenna.

[P.T.O]

24.	Explain the Ionosphere Abnormalities.

(OR)

25.	Derive (fc) critical frequency and maximum usable frequency (muf) of ionized layer.