Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	TRANSMISSION LINES AND WAVE GUIDES			      Time : 3 hours

Subject Code:	11EC212										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is an Infinite line?

2.	List the types of distortions in a transmission line.

3.	Nodes are the points where the magnitude of voltage or current is ______.

4.	When does a Quarter wave line behaves as a copper insulator?

5.	Define Attenuation factor.

6.	What are the properties of TEM waves in guided wave propagation?

7.	State the boundary conditions to be satisfied by Ez for TM waves in a rectangular wave guide.

8.	What is meant by intrinsic impedance?

9.	Explain what is the suitable mode used in circular wave guide for making a wave meter.

10.	How a waveguide becomes a cavity resonator?

PART – B  (5 x 3 = 15 MARKS)

11.	What is the transfer impedance of a transmission line?

12.	What is stub matching? What are the advantages of double stub matching over single stub matching?

13.	A parallel plate waveguide has plate separation 1cm and filled with a perfect dielectric of dielectric constant 9. Find the cut off frequencies of dominant and the next higher TM mode.

14.	What are degenerate modes and dominant modes in rectangular waveguide?

15.	A circular waveguide operated at 10 GHz has the internal diameter of 5.7 cm. For the TE01 mode propagation, calculate λ, λc and cut off frequency.

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the general solution of transmission line and explain the physical significance of it as an infinite line.

(OR)

17.	a.	What is meant by loading? Explain the different types of loading.					    (8)

b.	Find the sending end impedance of the line having Zo =710 ∟14o Ω, ZR = 300 Ω,                	γ = 0.007+j0.0028 / Km and the length of the transmission line is 100Km.			    (7)

18. 	a.	Explain a quarter wave transformer and its applications.							    (8)

b.	Determine the input impedance and SWR for a distance of 0.21λ from the load with a 	characteristic impedance of 50	Ω and a load impedance of 80 – j30 Ω.				    (7)

(OR)

19.	Consider a transmission line having characteristic impedance 75 Ω and a load of 150j225 Ω. Determine the location and length of short circuited stubs to match this line using Smith Chart.

20.	a.	Explain the characteristics of TE and TM waves.									    (8)

b.	Explain the different types of velocities of propagation in between two parallel plates.																						    (7)

[P.T.O.]

(OR)

21.	a.	Derive the expression for the field strengths for TM waves between parallel plates 	propagating in z direction. Sketch the field distribution of E and H fields for TM10 mode.																					  (10)

b.	If the plate separation is 10cm find the cut-off frequency and propagation constant at     	100 MHz for TM10 mode.														    (5)

22.	An air filled rectangular copper waveguide 0.9” x 0.4” is operated at 9.2GHz in dominant mode. Find Cut-off frequency, Guide wavelength, Phase velocity, Characteristic impedance and Conductor loss in dB. Also find the expression for the attenuation constant due to conductor loss and calculate the attenuation loss for 20cm length.

(OR)

23.	Derive the attenuation factor of TE waves in Rectangular wave guides.

24.	a.	Derive the expressions for TE wave components in circular waveguides using Bessel 			function.																		  (10)

b.	Write a brief note on excitation of modes in circular waveguides.					    (5)

(OR)

25.	Derive the expression for unloaded Q of a rectangular cavity resonator (a x b x d) excited in TE101 mode for d>a>b. Under what condition this Q is maximum and what is its expression?