Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	INDUCTION AND SYNCHRONOUS MACHINES		      Time : 3 hours

Subject Code:	11EE209										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention the two types of induction motors.

2.	Define slip.

3.	The starting winding of the single-phase induction motor is placed in the stator – True or False?

4.	How can we reverse the rotation of a single-phase, split-phase motor?

5.	Define regulation of an alternator.

6.	Mention the prime movers used for alternators.

7.	V-curve of a synchronous motor is the relation between __________ and __________.

8.	If the field of a synchronous motor is under-excited, the power factor will be __________.

9.	Define quadrature-axis reactance.

10.	A 50 Hz alternator will run at highest speed if wound for two poles – True or False?

PART – B  (5 x 3 = 15 MARKS)

11.	Define crawling and cogging with respect to three-phase induction motor.

12.	Why single-phase induction motor is non self starting?

13.	What are the conditions to be satisfied for the parallel operation of alternators?

14.	Discuss any one starting method for a synchronous motor.

15.	What is the frequency of voltage generated by an alternator having 10-poles and rotating at    720 rpm?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive an expression for the torque of an induction motor and obtain the condition for maximum torque.

(OR)

17.	A 3-phase, 400 V induction motor gave the following test readings:

No-load test			: 400 V, 1.25 kW, 9 A

Blocked-rotor test	: 150 V, 4 kW, 38 A

Draw the circle diagram. If the normal rating is 14.91 kW, find from the circle diagram, the full-load values of current, power factor, and slip.

18.	Describe double revolving field theory of single-phase induction motor.

(OR)

19.	Write a brief note on linear induction motor.

20.	Describe the constructional features of salient pole and non salient pole types of alternators.

(OR)

[P.T.O.]

21.	A 3-phase star-connected, 1000 kVA, 2000 V, 50 Hz, alternator gave the following open-circuit and short-circuit test readings:

| Field current (A) | 10 | 20 | 25 | 30 | 40 | 50 |
| --- | --- | --- | --- | --- | --- | --- |
| Open-circuit voltage (V) | 800 | 1500 | 1760 | 2000 | 2350 | 2600 |
| Short-circuit armature current (A) | ----- | 200 | 250 | 300 | ----- | ----- |

Draw the characteristic curves and estimate the full-load percentage regulation at 0.8 p.f lagging  and leading using MMF method. The armature effective resistance/phase is 0.2 Ω.

22.	Derive the expression for power developed by a synchronous motor.

(OR)

23.	Explain the phenomenon of hunting in synchronous motor and the methods of its suppression.

24.	Draw the phasor diagram of salient pole alternator under leading and lagging power factors based on two-reaction theory and explain.

(OR)

25.	A 5 kVA, 220 V, star-connected 3-phase salient-pole alternator with direct and quadrature axes reactances of 12 Ω and 7 Ω respectively, delivers full-load current at unity power factor. Calculate the excitation voltage, neglecting reactance.