Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRICAL MACHINE DESIGN					      	     Time : 3 hours

Subject Code:	11EE211										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the factors that decide the choice of Specific Electric Loading?

2.	Define Field Form Factor.

3.	What are the advantages of large length of air gap in DC machine?

4.	What is Square Pole and Square Pole Face?

5.	Distinguish between Core and Shell Type Transformer.

6.	Define Copper Space Factor.

7.	For a best power factor of a three phase induction machine, τ  is ___________.

8.	What is skewing?

9.	What is run away speed?

10.	List the factors to be considered for separation of D and L for salient pole machines.

PART – B  (5 x 3 = 15 MARKS)

11.	Define total gap contraction factor.

12.	Mention the guiding factors for the selection of number of poles.

13.	Why Circular Coils are preferred in Transformers?

14.	List out the methods to improve the power factor of the induction motor.

15.	How the value of SCR affects the design of  alternator?

PART – C  (5 x 15 = 75 MARKS)

16.	a.  	Derive the Output equation of a DC Machine.                                                                   (6)

b.  	A design is required for a 50 kW, 4 pole, 600 rpm, DC shunt generator, the full load terminal 	voltage being 220 V.  If the maximum gap density is 0.83 wb/m2 and the armature ampere conductors per metre is 30000, calculate the suitable dimensions of armature core to give a square pole face.  Assume that the full load armature voltage drop is 3% of the rated terminal voltage and the field current is 1% of rated full load current.  Ratio of pole arc to pole pitch is 0.67.                                                                                                       (9)

(OR)

17.	Calculate the apparent flux density at a section of the teeth of an armature of a dc machine from the following data at that section: Tooth width = 12 mm; slot width = 10 mm; length of armature core including 4 ducts of 10 mm each = 0.32 m; True flux density in teeth at that section is 2.2 wb/m2; permeability of teeth corresponding to real flux density = 31.4x10-6 H/m; stacking factor = 0.9.

18.	Find the armature diameter, core length, number of armature slots, armature conductors and commutator segments of a 350 kW, 440 V, 720 rpm, 6 pole DC generator with                         C0 =  163k VA/m3-rps and Bav = 0.61 wb/m2. Assume a square pole face with ratio of pole arc to pole pitch equal to 0.66.  The full load efficiency is 0.91 and the internal voltage drop is 4% of rated voltage.  The diameter of commutator is 0.7 of armature diameter.  The pitch of commutator segments should not be less than 4mm.  The voltage between adjacent segments should not exceed 15v at no load.

[P.T.O.]

(OR)

19.	A 50 HP, 4 pole, 480 V, 600 rpm, shunt motor has a wave wound armature with 770 conductors.  The leakage factor for the poles is 1.2.  The poles are to be of circular in cross section, the field coils are 70 mm thick and produce an mmf of 10000 A per pole.  The field density in the poles is 1.5 wb/m2.  Calculate the:

a.	diameter of pole 		b.	diameter of field wire 		c.	length of field coil

d.	turns per pole 		e.	field current.  Keep 20% of voltage applied to shunt field coil in reserve for speed control.  Assume that the thickness of insulation over wire is 0.125 mm; permissible loss is 1600 W/m2 of external cylindrical surface.  Resistivity of copper is 0.02 Ω/m and mm2.

20.	Determine the main and overall dimensions of the core, the number of turns and the cross section of the conductors for a 5 kVA, 11000/400 V, single phase core type distribution transformer.  The net conductor area in the window is 0.6 times the new cross section of iron in the core.  Assume a square cross section for the core, a flux density is 1wb/m2, a current density is 1.4 A/mm2 and a window space factor is 0.2.  The height of window is three times its width.

(OR)

21.	Design a suitable cooling tank with cooling tubes for a 1000 kVA, 6600/440 V, 50 Hz, 3 phase, delta/star, oil immersed natural cooled transformer.  The transformer tank is 1.9 m in height and 0.62 m X 1.52 m in plan. Core loss = 3.7 kW and I2R = 10.5 kW.  The average temperature rise of oil should not exceed 350 C. The specific heat dissipation from the tank walls is 6 W/m2-°C and 6.5 W/m2-°C due to radiation and convection respectively.  Assume that the convection is improved by 35% due to convection.

22.	Estimate the stator core dimensions, number of stator slots and number of stator conductors per slot for a 100 kW, 3300 V, 50 Hz, 12 pole star connected slip ring induction motor.  Assume: average gap density = 0.4 wb/m2, conductors per metre = 25000 A/m, efficiency = 0.9, power factor = 0.9, winding factor = 0.96.  Choose the main dimensions to give best power factor.  The slot loading should not exceed 500 A/m.

(OR)

23.	A 15 kW, three phase, 6 pole, 50 Hz squirrel cage induction motor has the following data:  Stator bore diameter = 0.32 m; axial length of stator core = 0.125 m; number of stator slots = 54; number of conductors per stator slot = 24; current in each stator conductor = 17.5 A; full load power factor = 0.85 lagging. Design a suitable squirrel cage rotor giving number of slots, section of each bar and section of each ring.  The full load speed is about 950 rpm.  Use copper for the rotor bars and end rings.  Resistivity of copper is 0.02 Ω/m and mm2.

24.	A 1250 kVA, three phase, 50 Hz, 3300 V, 300 rpm synchronous generator with a concentric winding has the following design data: Specific magnetic loading Bav = 0.58 wb/m2; specific electric loading ac = 33000 A/m; gap length = 5.5 mm; field turns per pole = 60; SCR = 1.2.  The effective gap area is 0.6 times the actual area.  Peripheral speed is 30m/s.  Find the stator core length, stator bore, turns per phase, mmf for air gap, armature mmf per pole and field current for no load and rated voltage.

(OR)

25.	The field coils of a salient pole alternator are wound with a single layer winding of bare copper strip of 30 mm deep, with separating insulation of 0.15 mm thick.  Determine a suitable winding length, number of turns and thickness of conductor to develop an mmf of 12000 A with a potential difference of 5 V per coil and with a loss of 1200 W/m2 of total coil surface.  The mean length of turn is 1.2 m.  The resistivity of copper is 0.02 Ω/m and mm2.