Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	POWER ELECTRONICS								      Time : 3 hours

Subject Code:	11EE212										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The Cut – in Voltage / Threshold Voltage of silicon diode is around ____________.

2.	The types of MOSFETs are ____________ and ____________.

3.	Define delay angle.

4.	In a single phase fully controlled bridge converter, the converter attains the inverter mode 	operation when the firing angle goes beyond ____________ degree.

5.	Write the expression for the average output voltage of a step-down chopper.

6.	The other name of ____________ is frequency converter.

7.	In a Current Source Inverter, if frequency of the output voltage is f Hz, then the frequency of voltage input to Current Source Inverter is____________.

8.	Inverters circuit uses forced commutation operation. (True or False)

9.	____________ is usually provided between gating circuits and power circuits for protection.

10.	What are advantages of SMPS over linear power supply?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the characteristics of IGBT.

12.	What is the purpose of free-wheeling diode in a circuit?

13.	What is commutation? List out its types.

14.	What is pulse width modulation? Mention its advantages.

15.	Explain fly back SMPS with relevant circuit diagrams.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Draw and explain the reverse recovery characteristics of a power diode.				   (7)

b.	Give the basic structure, characteristics and explain the four modes operation of a TRIAC.																					   (8)

(OR)

17.	a.	Compare Power MOSFET and Power BJT.										   (5)

b.	Discuss the static and switching operation of Thyristor.							  (10)

18.	Draw and explain the operation of single-phase Semi converter bridge rectifier with RL load with necessary waveforms. Also, derive the average and RMS output voltage of the rectifier.

(OR)

19.	 a. 	A single phase full converter bridge is connected to RLE load .The source voltage is 		230V, 50 Hz.The average load current of 10 A is constant over the working range for

R-0.4 ohm and L=2mH, Compute

i.	firing angle delay for E= 120 V.

ii.	firing angle delay for E= –120V.											  (10)

b.	Compare single phase full converter and semi converter.							    (5)

[P.T.O]

20.	Describe the operation of the single-phase ac to ac full wave voltage controller for a resistive (R) load with necessary circuits and waveforms. Derive the expression for the RMS load voltage and RMS load current.

(OR)

21.	a. 	Draw the circuit diagram of a single phase step-down cycloconverter. Mention the 	applications of a cycloconverter.												  (10)

b. 	A type – A chopper has Vdc = 230V, R = 10 Ohms. If the duty cycle is 0.4, calculate 	average 	voltage Vavg,  average current Iavg and output power Po.					    (5)

22.	Explain the 180 mode operation of a three-phase bridge inverter with an appropriate circuit diagram, waveforms and equations.

(OR)

23.	a.	Compare Voltage Source Inverter and Current Source Inverter.					    (5)

b.	Describe the operation of single phase series inverter with necessary circuit diagram and 	wave forms. 																	  (10)

24.	a. 	Explain Power converter control using Microprocessors with proper block diagram. 	    (8)	b.	Sketch the R-C phase shift control firing circuit for TRIAC based ac voltage controller 			using DIAC.																	    (7)

(OR)

25.	Draw the block diagram of various types of UPS and explain the functions of each block in them.