Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ENERGY SYSTEMS									      Time : 3 hours

Subject Code:	11EE242										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is meant by yaw control?

2. 	What is Photovoltaic effect?

3. 	Mention the name of the lowest level of tidal water.

4. 	List the main components of a fuel cell.

5. 	High frequency induction heating is used for ___________.

6.  	List the types of lighting systems.

7. 	List out the demerits of electric traction system?

8.  	Define tractive effort.

9. 	A Van de graff generator has a belt speed of 2.5 m/s. charge density of 10 μc/m2 and a belt width 2m. The maximum charging current is ____________.

10. 	Why we go for sphere gap measurement?

PART – B  (5 x 3 = 15 MARKS)

11.  	Mention the use of charge controllers.

12. 	What are the prospects of ocean thermal energy conservation (OTEC) in India?

13.  	List out the characteristics involved in good lighting scheme.

14. 	What is schedule speed of a train and list out its affecting factors?

15.  	How resistance shunt is usually designed?

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Explain the difference between HAWT and VAWT. 								    (7)

b. 	Describe the basic components of WECS. 										    (8)

(OR)

17. 	Explain with neat sketch the operation of solar PV system and discuss their applications.

18. 	With a neat sketch, explain the working of ocean tidal energy conversion system.

(OR)

19. 	a. 	Explain the principles of MHD power generation.								    (7)

b. 	Briefly explain fuel cells. 														    (8)

20. 	Explain in detail about the lighting design procedure for flood lighting.

(OR)

21. 	Discuss the various electric heating methods.

22. 	The distance between two stations is 1.6 kms and the average speed of a train is 40 kmph.the acceleration, retardation during coasting and breaking are 2kmphps. 0.16 kmphps and 3.2 kmphps respectively. Assume quadrilateral approximation of speed time curve. Determine the duration of the accelerating, coasting and breaking periods and distance covered during these periods.

(OR)

[P.T.O.]

23. 	Explain in detail about electric braking system.

24. 	With a neat sketch explain the working principle of Van de Graff generator.

(OR)

25. 	Explain the various high voltage measuring techniques.