Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DESIGN OF HEAT EXCHANGERS AND PRESSURE VESSELS FOR FOOD PROCESSING									     Time : 3 hours

Subject Code:	11ME205										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the term: overall heat transfer coefficient.

2.	What are the effects of fouling in a heat exchanger?

3.	What is the need for correction factor in Multi pass shell and tube Heat exchanger/?

4.	Why baffles are used in shell and tube heat exchangers?

5.	Draw the temperature distribution prevailing inside a condenser.

6.	Name at least two materials used for cooling towers.

7.	___________ air preheaters are also known as storage type preheaters.

8.	What is working principle of recuperative air preheaters?

9.	What is meant by multi-effect evaporator?

10.	An evaporator is also known as ____________coil.

PART – B  (5 x 3 = 15 MARKS)

11.	Define the term effectiveness of heat exchanger. Why effectiveness- NTU method is superior to LMTD method?

12.	Find the surface area required for a surface condenser dealing with 30000kg of steam per hour , 90% dry with latent heat of vaporization = 2440kJ/kg .The cooling water enters the condenser at 150C and leaves at 250C.Assume overall heat transfer coefficient as 3000 W/m2K.

13.	State the advantages of air cooled condenser over water cooled condenser.

14.	Explain the process of batch cooling of water with external heat exchanger using isothermal cooling medium.

15.	Define the capacity of evaporator. What are the modes of heat transfer possible in an evaporator?

PART – C  (5 x 15 = 75 MARKS)

16.	Find the length of the tube required for the following heat transfer where air is heated by exhaust gases. The inside and outside diameters of the tube are 5 and 6 cm respectively with heat transfer rate of 8000 W. The inside and outside heat transfer coefficient on air and gas side are 100 and 160 W/m2K respectively. Temperature are: Thi=35 0C,  Tho=150 0C Tci=50 0C and Tco= 100 0C.Neglect the tube  resistance and assume flow arrangement is parallel. If the flow is made counter then what is the percentage saving in tube length?						(8+7)

(OR)

17.	a.	A heat exchanger has 17.5 m2 area available for heat transfer. It is used for cooling coil at 	2000C by using water available at 20 0C.The mass flow and specific heat of oil are 	10000kg/hr and 1.9 kJ/kgK and the mass flow and specific heat of water are 3000kg/hr and 	4.18 kJ/kgK respectively. If the overall heat transfer coefficient is 300 W/m2k, estimate the 	outlet temperature of oil and water for parallel flow and counter flow arrangement by 	LMTD method.																  (10)

b.	Using Effectiveness –NTU method compare the heat transfer rate for counter flow 	configuration.																   (5)

[P.T.O.]

18.	a.	Fuel oil at the rate of 1.1kg/s is heated passing through the annulus of a counter flow 		double pipe heat exchanger from 100C to 200C by using hot water available from the 	engine at 710C. Water flows through a copper tube (OD =2.13 cm and ID=1.86 cm) with a 	velocity of 0.76m/s the oil passes to the annulus formed by inner copper tube and outer 	steel pipe(OD=3.34 cm and ID=3 cm).The fouling factor is 0.0004 m2 C/W and            	0.0009 m2 C/W on water and oil side respectively. Neglect the resistance of the copper 	tube.																		  (12)

| S.No | FLUID PROPERTY | OIL | WATER |
| --- | --- | --- | --- |
| 1 | Density (kg/m3) | 854 | 982 |
| 2 | Kinematic viscosity(m2/s) | 7.43x10-6 | 4.18x10-7 |
| 3 | Specific heat (J/kgK) | 1884 | 4187 |
| 4 | Thermal conductivity(W/mK) | 0.138 | 0.657 |

b.	Discuss the use of baffles in shell and tube heat exchangers and its types.			    (3)

(OR)

19.	a.	Draw the different configurations of shell and tube heat exchanger.					    (5)

b.	Draw the standard pattern of tube layout used in shell and tube heat exchanger. 		    (5)

c.	Why we need correction factor in the design of shell and tube heat exchanger? Explain. (5)

20.	a.	Discuss the design features of condenser in detail.								  (10)

b.	Explain in detail the pressure drop and Pumping power calculation in double pipe heat

exchanger. 																	    (5)

(OR)

21.	a.	Explain the working of mechanical draught cooling towers.						    (7)

b.	Explain the design steps involved in pressure drop and pumping power calculations in  			double pipe heat exchangers.													    (8)

22. 	Discuss in detail the working and design considerations for recuperative and regenerative air preheaters.

(OR)

23. 	From first principles, derive the temperature distribution for the heating and cooling of uniformly agitated batches. 													 (7.5+7.5)

24.	a.	Compare the single effect and multi effect evaporators. 							    (5)

b.	Explain in detail the working of multi-effect evaporators. Enumerate the criteria for 	selecting evaporator. 															  (10)

(OR)

25.	a.	What are the factors affecting the heat transfer capacity of an evaporator?			    (5)

b.	Explain the working of single effect evaporators.									  (10)