Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DIGITAL ELECTRONICS								      Time : 3 hours

Subject Code:	11MT210										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Convert the binary number (1101)2 into its gray equivalent.

2.	State De Morgan’s laws.

3.	List out any two applications of a multiplexer.

4.	Draw the logic diagram for a half subtractor.

5.	Write down the characteristic equation of a JK flip-flop.

6.	How many flip-flops are essential to construct a binary up counter that counts from 0 to 1023?

7.	Name the different types of Programmable Logic Devices.

8.	What are Configurable Logic Blocks?

9.	Draw the basic structure of a Moore Machine.

10.	What is the need for state reduction?

PART – B  (5 x 3 = 15 MARKS)

11.	Simplify the following Boolean expression to a minimum number of literals

(BC'+A'D)(AB'+CD').

12.	Design a three-bit even parity generator.

13.	Convert a D flip-flop into T flip-flop.

14.	Differentiate between fundamental mode circuits and pulse mode circuits.

15.	Draw the circuit for a CMOS two-input NAND gate and list out the characteristics of CMOS logic.

PART – C  (5 x 15 = 75 MARKS)

16.	Simplify the following Boolean expression F = m(1, 3, 7, 11, 15)+d (0, 2, 5) using  Karnaugh map in both sum of products and product of sum forms and implement the same using basic gates.

(OR)

17.	a.	Convert the decimal number (513.92)10 into its hexadecimal, octal and binary equivalent.								  														    (7)

b.	Explain in detail about single bit error detection / correction codes with a suitable example.							  														    (8)

18.	a.	Design a full adder using two half adders and suitable gates. Write down its truth table.

(8)

b.	Construct a 5 to 32 line decoder using four 3 to 8 line decoders with enable input and one 2 	to 4 line decoder.					   											    (7)

(OR)

19.	a.	Implement the boolean function F=m(1, 3, 7, 9, 12, 13, 15)	using a 8 : 1 multiplexer.								  														    (8)

b.	Design a four bit parallel binary adder / subtractor.		  						    (7)
																		[P.T.O.]

20.	Design a Mod-10 synchronous up counter using JK flip-flops.

(OR)

21.	Explain the working of a 4-bit Johnson counter with a suitable logic diagram and timing waveforms.

22.	Reduce the number of states in the following state table and tabulate the reduced state table.

| Present State | Next State |  | Output |  |
| --- | --- | --- | --- | --- |
|  | X=’0’ | X=’1’ | X=’0’ | X=’1’ |
| A | F | B | 0 | 0 |
| B | D | C | 0 | 0 |
| C | F | E | 0 | 0 |
| D | G | A | 1 | 0 |
| E | D | C | 0 | 0 |
| F | F | B | 1 | 1 |
| G | G | H | 0 | 1 |
| H | G | A | 1 | 0 |

(OR)

23.	a.	Differentiate critical races from non-critical races.		             					    (6)

b.	Explain the steps involved in the reduction of primitive flow table.  				    (9)

24.	Explain in detail about the operation of a TTL three-input NAND gate with a neat circuit diagram.

(OR)

25.	Discuss in detail about the architecture of a Field Programmable Gate Array with a neat block diagram.