Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	LINEAR INTEGRATED CIRCUITS						      Time : 3 hours

Subject Code:	11MT211										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	A differential amplifier amplifies the ___________ between two input signals.

2. 	The input offset voltage ___________  with negative feedback.

3. 	Basic differentiator fails to work at ___________ .

4. 	What are the applications of integrating circuit?

5. 	The OP-AMP comparator has ___________ .

6. 	The capture range of PLL is controlled by ___________ .

7. 	When PLL is used to demodulate the AM signal, it is locked to the frequency of      	___________ .

8. 	The conversion time of 16 bit successive approximation type ADC is driven by 1MHZ clock is	___________ .

9. 	The input stage of OP-AMP is usually a ___________ .

10. 	What is Switching Regulators?

PART – B  (5 x 3 = 15 MARKS)

11. 	Define slew rate.

12. 	Distinguish between input bias current and input offset current. State the reasons for the same.

13. 	What is meant by PLL?

14. 	What is a sample and hold circuit? Where it is used?

15. 	List the applications of 555 timer in monostable mode of operation.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe briefly about the input and output stages of an Op-Amp with the neat schematic diagram.

(OR)

17.	Explain the following :

a.	External frequency compensation.												    (8)

b.	Offset voltage applied to an  operational amplifier.  								    (7)

18. 	Explain the following Op-Amp Applications

a.	Integrator      	b.	Current –to_voltage converter								(8+7)

(OR)

19.	a.	Draw Op-amp Instrumentation amplifier and explain its principle.					    (8)

b.	What are Active filters? Design an Active filter which produces a rise in gain of 	40dB/decade before the corner frequency of 4kHz.  Explain with a frequency response of 	the same.																	    (7)

20.	Explain working of PLL using appropriate block diagram and explain any one application of the same.

[P.T.O.]

(OR)

21.	a.	List the desirable characteristics of VCO.										    (5)

b.	Explain the application of IC565 as FM demodulator.								  (10)

22.	Explain the R-2R ladder type DAC.

(OR)

23.	Explain how a comparator can be used as a zero crossing detector.

24.	Draw the block diagram of an Astable multivibrator using 555 timer and derive an expression for its frequency of oscillation.

(OR)

25.	Draw and explain the functional block diagram of a voltage regulator.