Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ANALOG ELECTRONIC CIRCUITS II					      Time : 3 hours

Subject Code:	11MT212										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define sensitivity of gain in feedback amplifiers

2.	Regarding the negative feedback in an amplifier. it ____________ gain stability .

3.	A circuit with an active device is used to produce alternating  current is called ____________.

4.	The total phase shift of an oscillator should be ____________. For feedback oscillator it should satisfies ____________ criterion.

5.	The amplifier with a circuit that is capable of amplifying a signal over a narrow band of frequencies are called ____________.

6.	The ratio of reactance to resistance is ____________.

7.	Name the types of Multivibrators.

8.	How many stable states do Bistable Multivibrator have?

9.	The period during which voltage increases linearly is called ____________ time in sawtooth generator.

10.	What is the other name of Astable blocking oscillator?

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the advantages of negative feedback over positive feedback?

12. 	Why is RC phase shift needed in a RC phase shift oscillator?

13. 	What are the advantages and disadvantages of tuned amplifiers?

14. 	How a Schmitt trigger is different from a Multivibrator?

15. 	Define blocking oscillator? Also list the applications of the same.

PART – C  (5 x 15 = 75 MARKS)

16.  	What is the effect of negative feedback on input impedance and output impedance of a current 	series  feedback circuit and draw the typical circuit of a current series feedback amplifier and 	explain.

(OR)

17. 	Show that in a feedback amplifier the effect of negative feedback provides stabilization of gain, reduction on distortion, increase input impedance and  decrease output impedance.

18. 	Describe the working of Colpitts oscillator and derive the expression for frequency of oscillation and condition for oscillation .

(OR)

19. 	Explain the working of a Wein bridge oscillator with relevant diagrams and derive an expression for the frequency of oscillation and condition for oscillation.

20. 	a.	Discuss the effect of cascading tuned amplifiers on its bandwidth.			       		    (5)

b.	Describe the operation of single tuned amplifier with neat sketches.                               (10)

(OR)

21. 	Derive the Efficiency of class C tuned amplifier and  also mention the applications of the same.

[P.T.O.]

22. 	Discuss with the help of waveforms, the operation of a clamping circuit to clamp a sine wave input  for the following

a.	positively at 0 V    		b.	negatively at 0 V

(OR)

23.	a.	With a neat sketch, explain the working of an Monostable Multivibrator.                         (7)

b.	Describe the working of a Schmitt trigger circuit with the help of necessary sketches.     (8)

24.	With neat sketches, describe the principle of operation of a Monostable blocking oscillator with emitter timing

(OR)

25.	Draw the circuit of a current-time base generator and explain its working with the necessary waveforms.