# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

# Subject Title: 	APPLIED ELASTICITY AND PLASTICITY				Time : 3 hours

# Subject Code: 	12CE302									         Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Derive the differential equations of equilibrium in two dimensions.				      (10)

b.	Given the following stress matrix in kN/m2, obtain the principal stresses and their direction cosines.

1020−4020−20−20−40−2010														      (10)

2.	a.	Discuss the methods of formulation of elasticity problems. 						(6)

b.	Explain the use of polynomials in the solution of rectangular beam problems. Using the stress function method, obtain the stresses in a cantilever beam fixed at x = 0; and carrying a concentrated load P at its free end. The beam is of rectangular cross section of width ‘b’ and depth ‘d’.													      (14)

(OR)

3.	Define

a.	Plane stress and plane strain problem  and 	b.	Airy’s stress function.			  (6+14)

Investigate what problem of plane stress  is satisfied by the stress function

Ф=3F4hxy−xy33h2+p2y2

applied to the region included in y = 0, y = h, x = 0 on the side x positive.

4.	a.	Define axisymmetric problem. Give examples of  Engineering problem for axisymmetric 	loading.																		(6)

b.	Discuss the problem of rotating disks of uniform thickness for a solid disc and for a 	circular disc with with a hole.												      (14)

(OR)

5.	a.	Discuss Winkler’s – Bach Theory stating the assumptions involved and its application.																				(6)

b.	Investigate the stresses in a circular disc under diametrical compression.		      (14)

6.	a.	Discuss the solution for the torsion problem of prismatic bars by displacement approach 	and 	force approaches. 														(6)

b.	An elliptical shaft of semi axes a=0.06 m, b= 0.03m and G= 80 GPa is subjected to a twisting 	moment of 1200π N-m. Determine the maximum shearing stress and the angle of twist per unit 	length. Derive the equations involved from fundamentals for torsion of elliptic section.															      (14)

(OR)

7.	a.	Explain how the membrane analogy can be applied to the torsional problems using the 		analogous values.															        (6)

[P.T.O.]

b.	Two thin walled rectangular tubes each of size 400 mm length and 320mm width are 			joined to form a single hollow multi-cellular rectangular tube with a common boundary. 		The thickness on 	the length side is 2 mm, thickness on the width side is 3 mm and 			thickness of the common boundary is  4mm. The cross-section is subjected to a torque of 		120 kN-m along the longitudinal axis. Determine the shearing stresses and the 				angle of twist. Assume G= 28 GPa.											      (14)

8.	a.	What do you understand by yield criteria? Explain any two yield criteria.		      (10)

b.	A rectangular beam having linear stress-strain behavior is 6 cm wide and and 8 cm deep. It is   3.5 m long, simply supported at the ends and carries a uniformly distributed load over the whole span. The load is increased so that the outer 2 cm depth of the beam yields plastically. If the yield stress of the beam material is 260 MPa, plot the residual stress distribution in the beam.												      (10)

(OR)

9.	a.	Write explanatory note on: Haigh-Westergaard stress space and Geometrical 	representation of 	yield criteria.												      (10)

b.	Explain the sand –heap analogy for solving plastic torsion problems. How this analogy is 		modified for sections having one or more holes? What are the limitations of this 				analogy?																      (10)