# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

# Subject Title: 	ADVANCED DESIGN OF REINFORCED CONCRETE STRUCTURES																		Time : 3 hours

# Subject Code: 	12CE303										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

(Use of IS codes may be permitted)

1.	Compulsory:

a. 	Differentiate between working stress, Limit state and ultimate load method.

b. 	Determine the nominal shear for the simply supported beam of a length of 5.25 m loaded from two columns at 1.75 m from each end with 3750 kN.  The total depth of beam is 4.2 m and width of the supports is 520 mm.  Also M40 grade of concrete and Fe415 steel are to be used.

2.	A flat plate with 7.5 m x 6 m panels on 500 mm x 500 mm columns has a slab thickness of     185 mm, designed for a total characteristics load (DL+ LL) OF 9.3 kN/m2. Check the safety of the slab in shear if 25 grade of concrete and Fe 415 steel are used for its construction. How can we increase the shear capacity of the slab?

(OR)

3.	a. 	Give the step by step procedure of interior panel slab and exterior panel slab.

b. 	A walkway consists of a slab 5.4m between edges supported on spandrel beams           200 x 600 mm in size, which in turn is carried on 300 x 200 mm columns spaced at 7m centre. Assuming that the total factored load on the walkway is 6 kN/m2 and the slab thickness is 150 mm, determine the design torsional moment in the spandrel beams and the walkway slab.

4.	a. 	Write the step by step procedure for design of a folded plates using beam method.

b. 	Write the step by step procedure for design of a folded plates using iteration method.

(OR)

5.	a. 	Write the step by step procedure for design of concrete shell with circular directrix using 	membrane theory.

b. 	Write the step by step procedure for design of concrete cylindrical shell using beam 	theory.

6.	a. 	Write the step by step procedure for design of square bunkers.

b. 	Give the step by step procedure of approximate analysis of grid floors.

(OR)

7.	Design a corbel to carry a factored load of 500 kN at a distance of 200 mm from the face of a 300 x 300 mm column.  Use M30 grade of concrete.

8.	A reinforced concrete slab is 105 mm thick with 20mm cover to centre of steel. If the positive steel reinforcement is 424 mm2/m, determine the approximate moment-curvature rotation diagram. Determine the ductility factor assuming M25 grade of concrete and Fe250 steel are used for construction.

(OR)

9.	a. 	Explain the concept of plastic hinges and Moment redistribution.

b. 	Give the step by step procedure of Moment redistribution of continuous beam.  .