# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination –  June 2014

# Subject Title: 	STABILITY OF STRUCTURES							Time : 3 hours

# Subject Code: 	12CE307								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

a.	Explain Euler’s theory of columns stability. Write its assumptions and limitations.   (10)

b.	Compare Rayleigh-Ritz method and Galerkin methods.						     (10)

2.	a.	A beam column subjected to a uniformly distributed load and an axial load is shown in 	figure. Obtain the expression for maximum deflection and maximum moment.  	     (15)

L/2

L/2

b.	Sketch two portal frames with and without sway.									(5)

(OR)

3.	a.	Determine the critical load of portal frame with sway shown in the figure using                    	equilibrium approach.   													     (15)

b.	Explain double modulus theory.												(5)

4.	a.	Explain torsional buckling and flexural buckling.								      (10)

b.	Discuss the procedure for the determination of double modulus for various sections.     	      																			      (10)

(OR)

5.	a.	A cantilever beam of uniform cross-section and length ‘l’ is subjected to a lateral load at 		the end through the centroid of the section. Find the buckling load.				      (15)

b.	Explain the energy approach for torsional buckling of thin walled open sections.		(5)

6.	a.	Derive the governing moment equilibrium equation for the buckling of a thin plate.   (10)

b.	Derive the critical load for plate in shear by Galerkin’s method.				      (10)

(OR)

7.	a.	Find the critical load of a rectangular simply supported plate of size (3a x a). The plate is 	loaded along its shorted edge by an udl of 5 kN/m.  Modulus of elasticity is 	0.7x105N/mm2 and poisons ratio is 0.3.   									      (15)

b.	Briefly explain the equilibrium method and energy concept of plates. 				(5)

[P.T.O.]

8.	a.	Derive Donnel’s equation for Symmetrical buckling of cylinder under uniform axial 			compression																      (15)

b.	Explain sensitive and insensitive systems.										(5)

(OR)

9.	a.	Explain perfect and imperfect systems.										     (10)

b.	Explain symmetric and asymmetric bifurcation with examples.					     (10)