# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

# Subject Title: 	ADVANCED DESIGN OF BRIDGES						Time : 3 hours

# Subject Code: 	12CE313							          		 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Design of R.C. Slab Bride for a national highway to suite the following data.

Carriage Width		- 8 m

Foot Path			- 1 m on either side

Clear Span			- 7 m

Wearing coat			- 100 mm

Width of bearing		- 300 mm2

Materials			- M40 and Fe415

Loading				- IRC class AA Tracked

2.	a.	Write the step for designing a Post – Tensioned Pre-Stressed Concrete Deck Slab.    (10)

b.	Discuss the major causes of prestressed bridge failures and indicate how these failures 	could be avoided.														      	     (10)

(OR)

3.	Design a Post – Tensioned Pre-Stressed Concrete Deck Slab Bridge for National Highways to suit the following data:

Clear Span					- 9 m

Clear Width of Road way		- 7.5 m

Foot Path					- 600 mm wide on either side

Thickness of wearing coat		- 100 mm at the center of the road

Live Load					- IRC Class AA Tracked Vehicle

Type of Construction – Class I

Material : M50 and 7 mm dia High strength Strand with ultimate tensile strength at          1500 MPa. The cable consists of 12 strands anchored at the end with a suitable diameter anchor block.  Compressive strength of concrete at transfer (fci= 40 MPa).  Loss Ratio – 0.8.

4.	Discuss the principles of Design of Cable stayed Bridges.

(OR)

5.	Write short notes on:

a.	Aerodynamic stability.														(5)

b.	Suspension bridge.															(5)

c.	Plate grinder.																	(5)

d.	Box grinder.																	(5)

6.	A well foundation is to be designed for an abutment of 10 m x 5 m base dimension.  The well is founded on a sandy soil, the date available are as follows:

Heights of bearing above the maximum scour level				- 28 m

Permissible Horizontal Displacement of the bearing level			- 50 m

Height of the abutment										- 6 m

Total vertical load including width of the abutment and well (considering Buoyancy effect) 20,000 KN.

Total lateral load at the scour level								- 400 KN

Submerged unit weight of soil.									- 9.5 KN/m3

Design the well and verify the stresses in the steining.

[P.T.O.]

(OR)

7.	a.	A foundation for substructures of a bridge consists of 16 piles to carry a total load of     	8000 KN.  The piles are spaced at 1.5 m c/c. They are driven through soft ground to a 	hard stratum available at a depth of 12 m.  Design the pile foundation using M20 	concrete and 	Fe415 Steels.													(15)

b.	Draw the typical Shapes of Piers.											(5)

8.	a.	Discuss different types of bearings with neat sketches.						(10)

b.	What are the common types of techniques used for bridge inspection.			(10)

(OR)

9.	a.	Explain Bridge maintenance management.									(10)

b.	Explain the following:

a.	Joints 		b.	Erection of precast elements.							(10)