# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

# Subject Title: 	HYDROLOGIC PROCESSES							Time : 3 hours

# Subject Code: 	12CE334								              	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain the structure and properties of water.									(5)

b.	Explain about Solar Radiation and triple cell circulation model.					(7)

(8)

c.	The evaporation from a lake is to be calculated by the water balance method. Inflow to the lake occurs through three small rivers, A, B and C. The outflow occurs through the river D. Calculate the evaporation from the lake surface during summer (May – August) if the water level was +571.04m on May1 and +571.10m on August 31. The lake surface area is 100 km2. The precipitation P during the period was 100 mm. Average inflows and outflow are given below:

River					Catchment (km2)					Average Q (m3/s)

A							150								15.0

B							120								20.0

C							130								17.0

D							---								45.0

2.	a.	Calculate the frequency factor for the normal distribution for an event with a return 	period of 50 years.															(5)

b.	The following data are the observed times between rainfall events at a given location.

Assuming that the inter-arrival time of rainfall events follows an exponential 	distribution, Determine the parameter λ for this process by the method of maximum 	likelihood. The times Between rainfalls (days) are: 2.40, 4.25, 0.77, 13.32, 3.55 and 1.37.

(15)

(OR)

3.	a. 	Explain the various types of distributions used to represent hydrologic variables and also 	the	suitability of these distributions for various hydrologic processes.			      (12)

b.	The annual peak discharge of a river follows the Gumbel’s distribution with a mean of 10,000 m3/s and a standard deviation of 3000 m3/s. What is the probability that the annual peak discharge is more than 15,000 m3/s? What is the magnitude of the peak discharge with an exceedence probability of 0.1?									(8)

4.	a.	How will you estimate precipitation using radar and satellites?						(5)

b.	Explain the various types of graphical representation of precipitation.				(7)

c.	Explain the various types of raingauges used for measuring precipitation.			(8)

(OR)

5.	a. 	Explain in detail about Depth Area Duration curves and its importance in hydrologic 	design.																		(5)

b.	Explain the various methods used for estimating missing rainfall data and also enumerate 		the 	benefits and limitations of each method.										(7)	c.	The normal annual rainfall at stations A, B, C and D in  a basin are 80.97, 67.59, 76.28 		and 92.01 cm respectively. In the year 1975, the station D was inoperative and the 			stations A,B and C recorded annual precipitations of 91.11, 72.23 and 79.89 cm 				respectively. Estimate the rainfall value at station D in that year.					(8)

[P.T.O.]

6.	a. 	Explain about the various basin characteristics which affects runoff.			      (12)

b.	Explain the various stages used for streamflow measurement.						(8)

(OR)

7.	a. 	Explain the various sources of streamflow and types of flow occurring on the surface due 	to this. 																	      (10)

b.	Analysis of data on a maximum one day rainfall  depth at Chennai indicated that a depth 	of 280mm had a return period of 50 years. Determine the probability of a one day rainfall 	depth equal to or greater than 280 mm at Chennai occurring a) Once in 20 successive 	years b) Two time in 15 successive years c) at least once in 20 successive years.	      (10)

8.	a.	Estimate the daily evaporation from a water surface using the Penman’s approach, at a 	place of Latitude 20 o N from the following data.							             (14)

Air temperature 							= 26 o C

Saturation vapour pressure at 26 o C			= 33.608 mb

Vapour pressure of air						= 13.25 mm of mercury

Number of sunshine hours					= 7.6

Maximum probable bright sunshine hours	=13.2

Solar radiation 							= 958 cal/cm/day

Wind velocity at 2 m height 				= 2.5 m/s

Atmospheric pressure						= 752 mm of mercury

b.	How will you construct a infiltration-capacity curve and explain its applications.		(6)

(OR)

9.	a. 	Explain the method of estimating evaporation by pan-evaporation method.		      (12)

b.	A 7-hour storm produced the following rainfall intensities (mm/hr) at half an hour intervals over a basin of area 1830 km 2

4, 9, 20, 18, 13, 11, 12, 2, 8, 16, 17, 13, 6 and 1

If the corresponding observed runoff is 36.6 million m 3, estimate Φ – index for the storm.																		(8)