Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	RENEWABLE ENERGY SOURCES						      Time : 3 hours

Subject Code:	12EE240										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is energy conservation?

2.	Give examples for secondary energy sources.

3.	What is the use of a photo voltaic system?

4.	Mention the important features of solar energy collectors.

5.	What is wind energy?

6.	The factor 0.593 is known as the _____________.

7.	Name any two resources of biomass.

8.	List out any two fuel properties of biomass.

9.	What is Ocean wave energy?

10.	Mention any two types of wave energy conversion devices.

PART – B  (5 x 3 = 15 MARKS)

11. 	Write short notes on PV hybrid system.

12. 	Wind speed is 10 m/s at the standard atmospheric pressure. Calculate:		a.	the total power density in wind stream	b.	the total power produced by a turbine of 100 m diameter with an efficiency of 40%. Air density=1.226 J/kg.K/m3.

13. 	What do you mean by biogas? Write a note on producer gas.

14. 	List some applications of fuel cells.

15. 	State the principle of operation of geothermal energy.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the principle and operation of a solar collector.

(OR)

17. 	Give a schematic diagram for a solar powered pump deriving its energy from a photo voltaic array. Provide a back-up storage battery in your circuit.

18. 	Draw a sketch of a horizontal axis single blade wind mill with details of transmission system and explain its working.

(OR)

19. 	Describe with a neat sketch the working of a wind energy conversion system (WECS) with its

main components.

20.	With a neat diagram discuss the biomass gasification method.

(OR)

21.	Discuss the method of power generation from liquid waste.

22.	Explain how hydrogen is a more versatile fuel. What are various methods of hydrogen storage?

(OR)

23.	Write short notes:		a.	Applications of hydrogen as fuel   	b.	Metal hydride.

24.	Discuss the global status vis-à-vis the current status of geothermal energy in India.

[P.T.O]

(OR)

25. 	Describe with the help of a simple schematic diagram the process of an ocean thermal energy conversion based on steam cycle.