# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

# Subject Title: 	DSP BASED CONTROL OF ELECTRIC DRIVES		Time : 3 hours

# Subject Code: 	12EE309										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1. 	Compulsory:

a.	Discuss Capture units and Quadrature Encoded Pulse (QEP) circuitry.			      (10)

b. 	Define interrupts. Explain Initializing and servicing interrupts in Software.		      (10)

2. 	Draw and explain the architecture of TMS 320 C24X Series of DSP controllers.

(OR)

3.	Write short notes on:

a.	Auxiliary register and data page pointer instructions.

b.	TREG, PREG, and multiply instructions.

c.	Branch instructions.

d.	Control instructions.

4.	Derive the expression for Clarke’s and Park’s transformation using DSP processor.

(OR)

5.	What are the types of modulation techniques? Explain Space Vector PWM Technique using DSP processor.

6. 	Derive the expression for Induction motor model in the arbitrary q-d-0 reference frame

(OR)

7.	Draw and explain the Field Oriented Speed Control of Induction Motor.

8.	What is the principle of operation of stepper motor? Discuss about the Implementation of DSP based Stepper Motor in detail

(OR)

9.	a. 	Draw and explain DSP based Switched Reluctance Motor drive.				      (10)

b.	Draw and Implement DSP based BLDC Motor Control.						      (10)