Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	FIBER OPTICS AND LASER INSTRUMENTATION		      Time : 3 hours

Subject Code:	12EI247											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the term acceptance angle of an optic fiber

2.	Suggest one method of overcoming signal dispersion.

3.	What are passive sensors?

4.	What is the sensor that is used in intensity modulated temperature sensor?

5.	Distinguish between stimulated and spontaneous emission very briefly.

6.	What will happen if all the modes are forced to maintain same relative phase to one another?

7.	Why is LIDAR referred as Laser Radar?

8.	Define the term trimming.

9.	Why is non destructive testing method preferred over destructive testing methods?

10.	What is the principle behind holograms?

PART – B  (5 x 3 = 15 MARKS)

11.	When the mean optical fiber launched into an 8 km length of fiber is 120µW, the mean optical power at the fiber output is 3µW. Determine the overall signal attenuation in dB and the signal attenuation per kilometer for the fiber.

12.	Brief on the principle behind fiber optic sensors used in industrial applications

13.	Mention all the properties of laser.

14.	What are the merits and demerits of using lasers for velocity measurement?

15.	What are conditions for recording and reconstruction of holograms?

PART – C  (5 x 15 = 75 MARKS)

16.	Categorize the types of light sources used and the major requirements on them to be used in fiber optic systems. Discuss in the detail the reasons. Why LED is preferred over lasers as optical source?

(OR)

17.	Explain in detail the construction, working, benefits and drawbacks of avalanche photodiodes.

18.	With a neat sketch explain the method of measuring fiber attenuation using optical domain reflectometer.

(OR)

19.	Give and explain the constructional block diagram and the response curves of fiber optic temperature sensor and displacement sensor.

20.	Explain the principle behind the production of laser beams. Classify lasers and explain the construction and working of semiconductor laser.

(OR)

21.	Explain the significance of the terms Q-switching and mode locking associated with lasers. How are they achieved?

[P.T.O.]

22.	Elaborate on the usefulness of laser in  various material processing techniques.

(OR)

23.	Discuss in detail the methods of measuring velocity and acceleration using lasers. Mention also the advantages and disadvantages of these methods.

24.	Why do lasers find wide application in medical field? Justify your answer with suitable explanation.

(OR)

25.	Detail on the usage of holography in non destructive testing methods.