# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

# Subject Title: 	WIRELESS SENSOR NETWORKS						Time : 3 hours

# Subject Code: 	12IT315					                 				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory :

- Discuss the challenges that necessitate energy awareness at all layers of a communications protocol stack in sensor network and also the key technology and standards elements that are relevant to accomplish them.		        				(6)

- Explain the architecture of sensor node with relevant sketch and narrate the functions of protocol stack of WSN.									      				      (14)

2.	On what basis the applications of WSN are categories into C1 and C2. Present your understanding by discussing any two applications in C1 category.

(OR)

3.	Applications in WSN are said to be reconfigurable. Justify this statement with suitable commercial application.  Also explain battery usage mechanisms adopted in sensor nodes to increase the longevity of application.

4.	State the functionalities that need to be supported in WSN design. Discuss the various Hardware and software subsystems used to achieve these functionalities with necessary diagrams.

(OR)

5.	Give a brief note on:

a.	Radio propagation modes.								       						(5)

b.	Modulation. 																	(5)

c.	Comparison of wireless technologies related to WSN.                                                  (10)

6.	Discuss the overview and various challenges involved in the design of Sensor – MAC and also highlight the key aspects in IEEE 802.15.4 and ZigBee reference model.

(OR)

7.	a.	Define data dissemination and gathering.						       				(6)

b.	Discuss the routing challenges and design issues in wireless sensor network.		     (14)

8.	Give a brief note on:

a.	Pump Slowly, Fetch Quickly. 								       					(5)

b.	GARUDA. 										       							(5)

c.	Ad Hoc Transport Protocol. 								  		                      (5)	d.	Reliable Multisegment Transport.									                      (5)

(OR)

9.	a.	How issues like naming, localization, maintenance, and fault tolerance are addressed by 	network management protocol in WSN. 								       		(8)

b.	Discuss any four operating systems used in WSN applications.					     (12)