# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

# Subject Title: 	DESIGN OF THERMAL POWER EQUIPMENTS		Time : 3 hours

# Subject Code: 	12ME303										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

The following readings were taken during a test on a surface condenser. Mean condenser temperature  is 35 °C, hot well temperature is 30 °C,  condenser vacuum is 69 cm Hg, barometer reading 76 cm Hg.  Condensate collected 60 kg/min.,  Cooling water enters at 20 °C and leaves atbn 32.5 °C, flow rate is 37500 kg/h. Calculate:

a.	Mass of air present per cubic meter of condenser. 	b.	Quality of steam at condenser inlet. 		c.	Vacuum efficiency. 		d.	Condenser efficiency.

2.	a.	Explain in detail the different type of furnaces used in Thermal Power Plants and list out 	merits and demerits of each.												      (16)

b.	What are the characteristics of an over fed fuel bed furnace? 						(4)

(OR)

3.	a.	List out the factors to be considered for the selection of a boiler and also discuss the 	essentials of a good steam boiler. 											      (16)

b.	What is the function of hot primary air? 											(4)

4.	a.	The following data refer to a boiler plant consisting of an economizer, a boiler and a 	super heater- Mass of water evaporated per hour = 5940 kg, Mass of coal burnt per 	hour=675 kg, L.C.V. of coal = 31600 kJ/kg, Pressure of steam at boiler stop valve = 14 	bar, Temperature of feed water entering the economizer = 32 °C, Temperature of feed 	water leaving the economizer = 115 °C, Dryness fraction of steam leaving the boiler and 	entering super heater is 0.96, Temperature of steam leaving the super heater =260 °C, 	specific heat of superheated steam =2.3. Determine:	i.	Percentage of heat in coal 		utilized 	in economizer, boiler and superheated.    ii.	Overall efficiency of boiler plant. 	      																		     (16)

b.	List the heat losses in a boiler plant. 											(4)

(OR)

5 . 	a.	Write brief notes on Indian Boilers Act.[IBR]. 								      (12)

b.	Define economizer. 															(4)

c.	What is desuperheater.														(4)

6 . 	a.	Explain Silica carry over gravity separation process with systematic diagram.            (16)

b.	Name any two chemicals in water purification. 									(2)

c.	What is steam distillation?														(2)

(OR)

7 . 	a.	Explain with neat sketch, the typical arrangements of a steam washing boiler drum and 	give its merits and demerits.												      (16)

b.	Write short notes on steam purification.											(4)

8 . 	a.	List out the types of air pre heaters and explain anyone of them in detail with a neat 	sketch and discuss application range of each.									      (16)

b.	Define forced draught.														(2)

c.	Write the expression chimney diameter.											(2)

(OR)

9 .	a.	Derive an expression for Chimney height and diameter and state the assumptions 	made.																	      (14)

b.	Write short notes on Electrostatic precipitator. 									(6)