# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

# Subject Title: 	ADVANCED STRENGTH OF MATERIALS				Time : 3 hours

# Subject Code: 	12ME307									      	 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	a.	A Cantilever 5 m long carries a uniformly distributed vertical load of 480 N per m from 	C to H and a concentrated load of 1000 N at its mid point. Construct the shear force and 	the Bending Moment Diagrams. 											(13)

b.	The stress at a point in a body is given by

σ=5323−10204Pa

Determine the normal strains and shear strains if Young’s Modulus E = 10 MPa and

ν = 0.33. 																	(7)

(OR)

2.  	For the stress matrix given below, determine the principal stresses and the directional cosines associated with the normals to each surface of the principal stress.

σ=311102120MPa

3.	Compulsory :

a.	The steel rods AD and CE shown in the figure below have a diameter of 10 mm. For 	steel let E = 200 GPa. The threads at the end of the rods are single threaded with a pitch 	of 1.5 mm. The nuts are first snugly fit with bar ABC horizontal. Next the nut at A is 	tightened one full turn. Determine the resulting tension in each rod and the deflection at 	points A and C if bar ABC is considered to be rigid. 							   (15)

ALL DIMENSIONS ARE IN MILLIMETERS

[P.T.O.]

b.	Explain in detail the Maximum Shear Stress Theory Criterion. 						(5)

4.	For the rod shown in the below figure, determine the deflection at point B due to the weight of the rod. For rod the mass density is ρ , area is A and the young’s modulus of the material is E. 																				(7)

b.	Explain Castigliono’s First Theorem with an illustrative example. 				(13)

(OR)

5. 	Two Cables shown in the figure are made from a non-linear material whose stress strain 	behavior is σ = Eɛ  Kɛ2, where E = 206 GPa, and K = (69,000) GPa. The Cross Sectional 	Area of each cable is (1.3*10-4)m2, and both have a length of L =1.5 m. Determine the 	vertical deflection δB of point B after a load of 8.9 kN is applied at B.

6.	Determine the maximum tensile stress and compressive bending stresses in the beam shown whose cross-sectional details are also given below.

(OR)

7.	A disk with radius r0 = 6.0 cm and ri = 1.0 cm is press fitted on to a shaft of radius 1.003 cm. Both members are steel with E = 30 Mpa and a Poisson ratio of ν = 0.29 and with weight densities of 0.284 kg/cm3 . Determine the stress distribution of the disk at n = 5000 rpm.

8.	Determine a Stress function for the stress field, for the given figure and evaluate the stress field.

(OR)

9.	a.	Explain Prandtls’ stress function for torsion.  									(7)

b.	A solid circular shaft of radius R0 is transmitting a torque T. Determine the corresponding shear stress distribution. 										      (13)