# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

# Subject Title: 	ADVANCED THERMODYNAMICS AND HEAT TRANSFER																				Time : 3 hours

# Subject Code: 	12ME311										 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Derive an expression for rate of heat transfer by conduction in a series of three 	compound resistances. 							          					      (10)

b.	Sketch and explain typical heat exchange equipment indicating its components.	      (10)

2.	a.	In a gas turbine unit, the gases flow through the turbine is 15 kg/s and the power 	developed by the turbine is 12000 kW. The enthalpies of gases at the inlet and outlet 	are 	1260 kJ/kg and 400 kJ/kg respectively, and the velocity of gases at the inlet and 	outlet 	are 50 m/s and 110 m/s respectively. Calculate: 		i.	The rate at which heat is 	rejected to the turbine		ii.	the area of the inlet pipe given that the specific volume of 	the gases at the inlet is 0.45 m3/kg.  					          				      (15)

b.	Define ‘internal energy’ and prove that it is a property of a system.					(5)

(OR)

3.	The internal energy of a certain substance is given by the following equation:

u = pv +84 where u is given in kJ/kg, p is in kPa and v is in m3/kg. A system composed of

3 kg of this substance expands from an initial pressure of 500 kPa and a volume of 0.22m3 to a final pressure 100 kPa in a process in which pressure and volume are related by

pv1.2 = constant.

a.	If the expansion is quasi-static, find Q, U and W for the process.

b.	In another process the same system expands according to the same pressure-volume relationship as in part (a) and from the same initial state to the same final state as in part (a) but the heat transfer in this case is 30 kJ. Find the work transfer for this process.

c.	Explain the difference in work transfer in parts (a) and (b).

4.	For a perfect gas, show that Cp−Cv=P+∂u∂VT∂V∂TP=ρυβ+υβ∂u∂vT  	where  is the co-efficient of cubical/volume expansion.

(OR)

5.	Determine the following when a gas obeys Vander Waal’s equation, p+av2v−b=RT

a.	Change in internal energy		b.	Change in enthalpy	c.	Change in entropy.

6.	a.	One kg of CO2 has a volume of 1 m3 at 1000 C. Compute the pressure by 	i.	Vander 		Waal’s equation 		ii.	Perfect gas equation. 									 (5)

b.	A container’s of 3 m3 capacity contains 10 kg of CO2 at 27oC. Estimate the 	pressure exerted by CO2 by using: 		i.	Perfect gas equation	ii.	 Vander waal’s equation,

iii.	 Beattie Bridge equation.					           						      (15)

(OR)

[P.T.O.]

7.	a.	1 kg of air at a pressure of 8 bar and a temperature of 1000C undergoes a reversible 	polytrophic process following the law pV1.2 = constant. If the final pressure 	is 1.8 bar 	determine: 	i.	the final specific volume, temperature and increase in entropy,

ii.	the work done and the heat transfer. Assume R = 0.287kJ/kg K and  = 1.4.

b.	Repeat (a) assuming the process to be irreversible and adiabatic between end states.

8.	Prove that the mean temperature difference in a parallel flow heat exchanger is given by

LMTD (tm) =t1−t2loget1t2

(OR)

9.	a.	Exhaust gases flowing through a tubular heat exchanger at the rate of 0.4 kg/s are cooled 	from 450o C to 150oC by water initially at 15oC. The specific heat of exhaust gases and 	water may be taken as 1.13 and 4.19 kJ/kgK respectively, and the overall heat transfer 	co-efficient from gases to water is 140 W/m2K. Calculate the surface area required 	when the cooling water flow is 0.5kg/s:		i.	for parallel – flow	ii.	for counter – 	flow. 								          							              (15)

b.	State the laws of radiation. 													(5)