Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	TRANSMISSION LINES AND WAVE GUIDES			      Time : 3 hours

Subject Code:	EC280											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How much inductive loading is required to make a 16 gauge cable distortion less? The line parameters are R = 42.1 Ω/km, G = 1.5 μmho/km, C = 0.062 μF/km and L = 1 mH/km.

2.	State the relationship between Group and phase velocity.

3.	Define SWR.

4.	Why Quarter wave line is called as copper insulators?

5.	List any one property of TEM waves in parallel plane guides.

6.	A parallel plate waveguide has plate separation 2 cm and is filled with a perfect dielectric of dielectric constant 4. The cut off frequency of dominant TM mode is __________.

7.	A rectangular waveguide is a _________ filter.

8.	State the reason for the non existence of TM01 and TM10 modes in a rectangular waveguide.

9.	What is cut-off wave number?

10.	Which cavity resonator is used in microwave frequency meter?

PART – B  (5 x 3 = 15 MARKS)

11.	Calculate the reflection coefficient of a transmission line when ZR = 200 Ω and

Zo = 745ے12o Ω.

12.	Design a Quarter wave transmission line to match a load of 900 Ω to a source of 100 Ω. The operating frequency is 300 MHz.

13.	What is attenuation factor in Waveguides? Write its expression.

14.	What are degenerate and dominant modes in a rectangular waveguide?

15.	Define Quality factor of a resonator. What is the relationship between loaded and unloaded Q?

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the general solution of transmission line in two different forms and state the physical significance of an infinite line.

(OR)

17.	a.	Explain the different types of waveform distortions in transmission lines. 			    (5)

b.	Derive the condition of a distortion less line. 									  (10)

18.	a.	Derive the equations of the circle diagram for the dissipation less line. 				    (8)

b.	Determine the input impedance and SWR for a distance of 0.21 λ from the load with a 	characteristic impedance of 50	Ω and a load impedance of 80 – j30 Ω.				    (7)

(OR)

19.	A line of characteristic impedance Ro = 600 Ω is connected to a load of 100 Ω. For a frequency of 100 MHz, find the length and location nearest to the load of a single stub to produce an impedance match.

[P.T.O.]

20.	Derive the expression for the field components of TE waves in Parallel plane waveguide. Sketch the field distributions for the dominant mode.

(OR)

21.	Prove that the frequency of minimum attenuation due to conductor loss in a parallel plane waveguide for TM mode is √3 fc where fc is the cut off frequency. Also find the expression for minimum attenuation.

22. 	A Rectangular air filled wave guide with dimension 6.5 x 3.5 cm cross section is operated at 3 GHz with the dominant mode. Find the phase constant, guide wavelength, phase velocity, group velocity and wave impedance.

(OR)

23.	a.	Prove that TEM waves do not exist in hollow waveguide.							    (7)

b.	Describe the excitation methods for different TE and TM waves in rectangular waveguide.																					    (8)

24.	Derive the expression for unloaded Q of a rectangular cavity resonator (a x b x d) excited in TE101 mode for d > a > b. Under what condition this Q is maximum and what is its expression?

(OR)

25.	a.	Derive the expressions for cut-off frequency, phase constant and velocity of propagation of 	waves in a circular waveguide for TEnm modes.									    (7)

b.	Given is a circular waveguide of internal diameter 2 cm operating with a 10 GHz signal 			propagating in dominant mode. Calculate the cut-off frequency, guide wavelength, wave 			impedance and the bandwidth of operation in dominant mode only.

[(ha)’11 = 1.841].																    (8)