Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRON DEVICES									      Time : 3 hours

Subject Code:	EC283											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The band gap of silicon at room temperature is ___________.

2. 	Free electrons available in the ____________ band.

3. 	The built in potential of the junction is ___________.

4. 	In a pn junction diode under reverse bias the magnitude of electric field is maximum at ___________.

5. 	A transistor for which the region is very lightly doped and is very thin is called as _________.

6. 	Arrow head on a transistor symbol indicates __________.

7. 	FET has __________ input impedence.

8. 	UJT  has a __________resistance region.

9. 	Which diode can be used as a regulator?

10.	What is the input to the base of photo transistor?

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain the combined effect of electric and magnetic field.

12. 	Define the term diffusion capacitance.

13. 	Draw the input and output characteristics of a transistor in CE configuration and mark the

Cutoff, saturation and active regions.

14. 	Compare JFET and MOSFET.

15. 	What is a tunnel diode? Mention its important applications.

PART – C  (5 x 15 = 75 MARKS)

16.	Obtain the relation for Electrostatic deflection sensitivity.

(OR)

17.	Explain the Force on a particle and motion of a particle in magnetic field.

18.	Explain the following

a. 	Mobility 				b. 	Drift current 			c. 	Conductivity

d. 	Diffusion current			e.	Hall effect

(OR)

19.	Derive the expression for the total current in PN junction diode.

20.	a.	Compare the performance of  CE, CB  and CC configuration.						    (5)

b.	Draw the input and output characteristics of CE configuration.					          (10)

(OR)

21.	With a neat diagram explain the operation of a BJT and derive all the current components and emitter injection efficiency.

22.	With necessary diagram explain the junction FET operation and its static characteristics.

(OR)

[P.T.O.]

23.	Draw the equivalent circuit of UJT and explain its operation with the help of emitter Characteristics.

24.	a.	Explain VI characteristics of the zener diode. 									    (8)

b.	What is break down in zener diode and explain its types?							    (7)

(OR)

25.	a.	List out the advantages and disadvantages of LCD.								    (5)

b.	Describe with the help of a relevant diagram, the construction of an LED and explain its 	working.																		  (10)