Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DIGITAL ELECTRONICS								      Time : 3 hours

Subject Code:	EC284											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Convert decimal 153 into octal equivalent.

2.	How many two-input AND and OR gates are required to realize Y = CD+EF+G?

3.	Mention the universal gates.

4.	Mention the number of control lines for 32 to 1 multiplexer.

5.	Write down the characteristics equation for JK flip-flop.

6.	How many flip flops are required to construct a decade counter?

7.	How many flip flops are required to count 16 clock pulses?

8.	When an input signal A=11001 is applied to a NOT gate serially, mention its output signal.

9.	Which TTL logic gate is used for wired ANDing?

10.	Mention the digital logic family which has the lowest propagation delay time.

PART – B  (5 x 3 = 15 MARKS)

11.	Apply De-Morgan’s theorem to the expression Y = ((A +B+C)´D´).

12.	Obtain 3-level NOR-NOR implementation of f (a, b, c, d, e, f) = (ab + cd) ef.

13.	How to convert JK flip flop into T flip flop? Draw the circuit diagram. Write the truth table.

14.	List the steps involved for state reduction.

15.	Draw the circuit of a HTL NAND gate.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	List the salient features of the Gray codes.										    (6)

b.	Reduce the following expression using 5 variable K-map:

f(A, B, C, D, E) = ∑ m (0, 1, 2, 3, 10, 12, 13, 14, 20, 21, 22, 23, 26, 27, 28, 29, 30, 31)

(9)

(OR)

17.	Find the minimal expression for the following function using Quine- McCluskey method.

f(A, B, C, D) = ∑ m (0, 1, 4, 6, 8, 9, 10, 12) + d(5, 7, 14)

18.	Design an excess-3 to BCD code converter.

(OR)

19.	a.	Design a 4-bit adder using three full adders and one half adder.					    (7)

b.	Obtain a 1:4 line demux using 1:2 demultiplexers.								    (8)

20.	By using suitable flip flops design a counter to go through the following states.

0 – 2 – 4 – 6 – 7 – 0

Avoid lockout conditions.

(OR)

[P.T.O.]

21.	Design a Modulo-5 ripple counter using a 3-bit ripple counter.

22.	For the given state table do the state reduction and realize using flip flops.

| Present State | X = 0 |  | X = 1 |  |
| --- | --- | --- | --- | --- |
|  | Next state | Output | Next state | Output |
| a | a | 0 | b | 0 |
| b | c | 1 | d | 0 |
| c | a | 1 | - | - |
| d | e | 1 | f | 0 |
| e | g | 1 | - | - |
| f | a | 1 | - | - |
| g | e | 1 | - | - |

(OR)

23.	Design a sequential circuit using Mealy machine to detect an overlapping sequence as follows: …..1010…..

24.	a.	Explain the working of DTL gate.												    (7)

b.	Explain the working of three input ECL NOR gate.								    (8)

(OR)

25.	a.	Explain with a circuit the working of three state TTL gates.						    (7)

b.	Illustrate the operation of CMOS NAND and NOR gates with the circuit and truth table.

(8)