Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	SIGNALS AND SYSTEMS								      Time : 3 hours

Subject Code:	EC285											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	x(t) = exp(j5t) is a periodic signal . True / False.

2.	y(t) = 2tx(t–1)  is ________system. (time invariant / time variant ).

3.	Any periodic continuous time signal has___________ frequency spectrum. (continuous/discrete).

4.	In a CT-LTI system, Fourier transform of impulse response gives frequency response. True / False.

5.	Nyquist sampling rate for a signal with maximum frequency of 10 KHz is ______samples/sec.

6.	Laplace transform is a tool to analyze _________ time system. (continuous/discrete).

7.	The spectrum of non periodic discrete time signal is continuous. (True/False).

8.	Discrete time signals are modeled as ________equation. (differential / difference).

9.	The poles of Transfer function of a stable DT-LTI system, lie outside the unit circle in z plane. True / False.

10.	Transfer function and impulse response of a DT-LTI system, form a Z transform pair. True /False.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the condition for causality and stability of a DT-LTI system.

12.	Write the convolution property of Fourier Transform.

13.	State low pass sampling theorem.

14.	State Parseval’s energy theorem for discrete time signal.

15.	Find the Z transform of u(n–1).

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Define the following signals. 													(3x3)

i.	Unit impulse signal.

ii.	Energy signal.

iii.	Power signal.

b.	Sketch the following signals. 													(2x3)

i.	4u(t–2)     		ii.	r(–t+3)

(OR)

17.	For the systems shown below check for (a) Linearity (b) Time invariance (c) Stabilty (d) Causality							        										(7+8)

i.	y(t) = 2x(t)+3x(t–1)	  		ii.	y(n) = n sin(x(n))

18.	Obtain the Fourier Transform of the signal shown below.

x(t) = A  ;   –T/2  ≤  t  ≤  T/2

=  0  ;    elsewhere.

(OR)

[P.T.O.]

19.	Determine:	a.	Unit impulse response		b.	Frequency response of the CT-LTI system 		defined by d2y/dt2 + 5dy/dt + 6y(t) = dx/dt + x(t). 									(8+7)

20.	Explain the low pass sampling theorem with necessary diagrams.

(OR)

21.	Give Inverse Laplace Transform of the functions given below.

a.	Xs=ss+1			b.	Xs=ss2+5s+6.								(7+8)

22.	Write any three properties of DTFT and prove the same.            		     				(3x5)

(OR)

23.	A nLTI system is given with the impulse response h(n) = (0.5)n u(n). Determine Frequency response and hence plot the same.				     									(8+7)

24.	Find the Z transform of the following.

a.	 xn u(n)				b.	n u(n)												(7+8)

(OR)

25.	A discrete time system defined by y(n) + 0.2 y(n–1) = x(n). Determine its

a.	Transfer function H(z)

b.	Impulse response h(n).

c.	Unit step response s(n). 													   (5+5+5)