Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	SOLID STATE CIRCUITS								      Time : 3 hours

Subject Code:	EC288											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a rectifier?

2.	What is the maximum efficiency of a FWR?

3.	Define operating point.

4.	What is thermal runaway?

5.	What is frequency response curve of an amplifier?

6.	Classify power amplifiers on the basis of mode of operation.

7.	The gain of an amplifier with negative feedback is known as ____________ gain.

a.	Closed loop            		b.	Open loop

c.	Resonant                          	d.	none of the above

8.	What is meant by CMRR of a differential amplifier?

9.	Response of tuned amplifier is ____________ at resonant frequency and it ____________ sharply for frequencies below and above the resonant frequency.

10.	List the substances which show piezoelectric effect.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the principle of operation of a half wave rectifier circuit.

12.	List the various biasing circuits for BJT and sketch the voltage divider bias circuit for a BJT.

13.	With a neat diagram explain the operation of a class A amplifier.

14.	The voltage gain of a certain amplifier without feedback is 400. If the feedback ratio is equal    to 0.1, find the voltage gain of the amplifier with feedback.

15.	List the advantage and disadvantage of tuned amplifier.

PART – C  (5 x 15 = 75 MARKS)

16.	A full wave rectifier uses RL=500 ohms. Each diode is considered to have idealized characteristics with Rf =5 ohm, Rr= ∞; Vm >> Vγ; The sine wave ‘V’ applied has amplitude of 15V and frequency 50 Hertz. Calculate peak RMS, DC load current, DC power of input and output, efficiency, form factor and ripple factor.

(OR)

17.	Explain the operation of capacitor filter and derive its ripple factor.

18.	a.	Define stabilization and mention the need for stabilization.

b.	Define stability factor and derive the expression for it and hence derive the stability factor for fixed bias circuit.

(OR)

19.	Explain the various biasing methods for FET.

20.	Explain the principle of operation of RC coupled amplifier and draw its frequency response curve and explain the reason for various slopes.

(OR)

[P.T.O.]

21.	Describe the operations of transformer coupled push pull class B power amplifier and mention its advantages and disadvantages.

22.	Mention the characteristics of negative feedback connections and explain in detail.

(OR)

23.	Explain the following terms of differential amplifier: output offset voltage, input offset voltage, common mode voltage gain, differential mode voltage gain and common mode rejection ratio.

24.	Draw the circuit diagram of Hartley and Colpitts oscillator and explain and obtain the expression for frequency of oscillation.

(OR)

25.	Draw the circuit diagram of single and double tuned amplifier and their frequency response and explain.