Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRIC CIRCUITS									      Time : 3 hours

Subject Code:	EE245											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	An ideal voltage source should have ___________(zero/infinite) internal resistance.

2.	If two voltage sources each of voltage ‘V’ are connected in parallel the equivalent voltage of voltage sources is 2V. True/False

3.	For a sine wave, RMS value = 0.707 times its peak value. True/False

4.	Power factor tells us about phase difference between voltage and current. True/False

5.	KVL is the basic law behind mesh analysis. True/False

6.	In nodal analysis, any node can be taken as a reference node. True/False

7.	While finding Thevenin’s resistance, the current sources should be replaced as __________ (open/short) circuit.

8.	For maximum power transfer from source to load, load resistance should be equal to ___________.

9.	At resonance, given RLC circuit behaves like a pure resistive circuit. True/False

10.	Quality factor is inversely proportional to bandwidth. True/ False

PART – B  (5 x 3 = 15 MARKS)

11.	State Kirchoff’s Laws.

12.	Define Form and Peak factors.

13.	Why do we go for mesh method of solving problems?

14.	State superposition theorem.

15.	Define resonance in a RLC circuit.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the current in 10 Ω resistor shown in the figure.

(OR)

[P.T.O.]

17.	Obtain the equivalent resistance between terminals A and B in figure.

18.	If v(t) = 50 sin(314t+45) volts, applied across a resistance of R = 10Ω, determine:

a.	Current in R		b.	Frequency in Hz		c.	Phase angle between current

d.	Power dissipation in R 		e.	Power factor.

(OR)

19.	Determine the Average and RMS values of the waveform shown in the figure.

20.	Using mesh method, find the current in 12 Ω resistor in the figure.

(OR)

21.	Using nodal method, find the voltage across 5 Ω resistor in the figure.

22.	Obtain the Thevenin’s equivalent circuit across AB in the figure.

(OR)

23.	With an example state and prove superposition theorem.

24.	Derive the resonant frequency for the circuit shown in the figure.

(OR)

25.	A series RLC circuit has R = 100 Ω, L = 200 milliH and C = 10 microF with applied  voltage of 10V. Determine:

a.	Resonant frequency		b.	Current at resonance		c.	Lower and upper cutoff frequencies 			d.	Bandwidth.