Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	NETWORK THEORY									      Time : 3 hours

Subject Code:	EE246											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is the phase angle between inductor current and the applied voltage in a parallel RL circuit?

2. 	In a three phase unbalanced star connected system, the vector sum of the currents in the three lines is _____________.

3. 	What is the maximum value of the coefficient of coupling?

4. 	Write two applications of tuned circuit.

5. 	What is the time constant of a series RL circuit?

6. 	Transient current in an RLC circuit is oscillatory when _____________.

7. 	The h parameters h11 and h12 are obtained by _____________.

8. 	The attenuation of a wave filter can be expressed in _____________.

9. 	In the first Foster form, the presence of first element capacitor indicates

a.	pole at ω=0 		b.	pole at ω=infinity 	c.	zero at ω=0 		d.	zero at ω=infinity

10. 	Write the basic difference between network analysis and network synthesis.

PART – B  (5 x 3 = 15 MARKS)

11. 	What are the methods used to measure the power in three phase circuit? Draw any one method’s circuit diagram for a star connected load.

12. 	Write the characteristics of an ideal transformer.

13. 	Write the significance of Poles and Zeros?

14. 	Design a π-type attenuator to give 20dB attenuation and to have a characteristic impedance of 100Ω.

15. 	State any three properties of Hurwitz polynomials.

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	A three phase system with an effective voltage of 400V, has a balanced load with 	impedances 3+j4Ω. Calculate the:	i.	phase currents 	ii.	line currents.                   (8)

b.	Two watt meters are connected to measure power in a three phase circuit. The reading of the one wattmeter is 5 kW when the load power factor is unity. If the power factor of the load is changed to 0.707 lagging, without changing the total input power, find the readings of the two watt meters.                                                                                                  	    (7)

(OR)

17.	a.	A delta connected three phase load has 10 Ω between R and Y, 6.36 mH between Y and B 	and 636μF between B and R. The supply voltage is 400V, 50 Hz. Calculate the line 	currents for RBY phase sequence.							          				    (8)

b.	A balanced delta connected load of (2+j3) Ω per phase is connected to a balanced three 			phase 440 v supply. The phase current is 10A. Find the:

i.	total active power 	ii.	total reactive power 		iii.	apparent power.  		    (7)

[P.T.O.]

18. 	a.	Derive the equivalent inductance of parallel connection of coupled inductors with a neat 	diagram. 				           						           					  (12)

b.	Two coils connected in series have an equivalent inductance of 0.4 H when connected in 	aiding, and an equivalent inductance 0.2 H when the connection is opposing. Calculate the 	mutual inductance of the coils. 													    (3)

(OR)

19.	a.	An iron of 10 cm diameter and 15 cm2 cross sectional area is wound with 250 turns of wire 	for a flux density of 1.5 Wb/m2 and permeability is 500. Find the exciting current, the 	inductance and stored energy. Find all the corresponding quantities when there is a 2 mm 	air gap. 	                                                                                                                       	  (10)

b.	Two inductively coupled coils have self inductances L1=50mH and L2 = 200mH. If the 			coefficient of coupling is 0.5, 		i.	find the mutual inductance between the coils,

ii.	what is the maximum possible mutual inductance?							    (5)

20.	a.	Derive the Transient response of RLC Series circuit for step input. 	           			  (10)

b.	In the figure shown below, assume that all the capacitors are initially uncharged. If 				Vi(t)=10u(t) volts. Find V0(t).													    (5)

(OR)

21.	a.	In the circuit shown, the switch S is open for a long time and is closed at t=0. Find the 			current i(t) for t≥0+.  	 														    (8)

b.	Derive the Transient response of RL Series circuit for a step input.  		            	    (7)

22.	a.	The impedance parameters of a two port network are Z11=6Ω; Z22 = 4Ω; Z12=Z21=3Ω. 			Compute the Y parameters and ABCD parameters and write the describing equations. 	                                                                                                              																					    (8)

b.	Design and draw a high pass filter having a cut off frequency of 1kHz with a load 				resistance of 600Ω. 															    (7)

(OR)

23.	a.	Design a symmetrical lattice attenuator to have a characteristic impedance of 800Ω and 	attenuation of 20dB. 															    (7)

b.	Find the impedance parameter Z11 and Z12 of the given two port network.  			    (8)

24. 	The driving point impedance of a one port reactive network is given by,

Z(s)=5S2+4S2+25sS2+16.Obtain the first and second Foster networks.

(OR)

25.	Find the R-C network corresponding to the driving point impedance function.

Zs=s+4s+8s+2s+6.