Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRO MAGNETIC FIELDS						      Time : 3 hours

Subject Code:	EE248											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a unit vector?

2.	∇×F=__________.

3.	Write the unit of Electric filed intensity.

4.	∇2V = _________.

5.	Write the unit of current density.

6.	State point form of ohm’s law.

7.	Write the Maxwell’s equation from faraday’s law in Integral form.

8.	State Lenz’s law.

9.	Define TEM wave.

10.	Write the expression for phase velocity.

PART – B  (5 x 3 = 15 MARKS)

11.	Write down the expression for x, y, z in terms of spherical coordinates r, θ and Φ.

12.	State Coulomb’s law of force between any two point charges and state the units of force.

13.	State and explain Biot-Savart law.

14.	Explain the concept of Displacement current.

15.	Write the properties of uniform plane wave.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	State and prove Stokes theorem with an example.					    				  (12)

b.	Find the unit vector directed towards the point (x1, y1, z1) from an arbitrary point in the plane y= –5.										   							    (3)

(OR)

17.	a.	Explain various coordinate systems in electromagnetic theory.			  			  (12)

b.	State divergence theorem.						                          					    (3)

18.	a.	State and prove Gauss’s law and write any one application of Gauss Law.           		  (12)

b.	What is meant by equipotential surfaces?                                                              		    (3)

(OR)

19.	a.	Derive an expression for electric field intensity E due to an uniformly charged infinitely 	long straight line with constant charge density in C/m.                        				  (10)

b.	A line charge of 20 nC/m is located at x=2m and y=-4m. Calculate the field E at       
      		(–2, –1,4) m.                                                                                                            		    (5)

20.	a.	Obtain the expression for magnetic field intensity in all the regions if a cylindrical 	conductor carries a direct current I and its radius ‘R’ m. Plot the variation of magnetic field 	intensity against the distance r from the center of the conductor.					  (10)

b.	State and prove Ampere’s circuital law.											    (5)

[P.T.O.]

(OR)

21.	a.	Define inductance and derive an expression for the inductance of solenoid.          	  (10)

b.	Calculate the inductance of a solenoid of 200 turns wound tightly on a cylindrical 
         	tube of 6 cm diameter. The length of the tube is 60 cm and the solenoid is in air.  	    (5)

22.	a.	Derive the Maxwell’s equation in both integral and differential form.                     	  (12)

b.	State Faraday’s law of electromagnetic induction.                                                  		    (3)

(OR)

23.	Explain following terms:

a.	Transformer and Motional e.m.f.                                                                                    (6+6)

b.	Eddy current.                                                                                                          		    (3)

24.	a.	Derive the electromagnetic wave equation.                                                             		    (7)

b.	A 300 MHz uniform plane wave propagates through fresh water for which σ =0,  μr=1, and   εr =78. Calculate:

i.	The attenuation constant		ii.	The phase  constant		iii.	The wave length  iv.	Intrinsic impedance.			           								    		    (8)

(OR)

25.	a.	Explain the wave propagation in Lossy dielectrics.                                               		    (7)

b.	State and prove Poynting Theorem.                                                                       		    (8)