Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DC MACHINES AND TRANSFORMERS				      Time : 3 hours

Subject Code:	EE250											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	_______ losses are reduced by laminating the core of a dc generator.

2.	_______ is the insulating material normally used in between the commutator segments.

3.	The condition for maximum efficiency for a dc generator is
(a)   Eddy current losses= Stray losses	(b)    Hysteresis losses = Eddy current losses

(c)   Copper losses = 0   				(d)    Variable losses = Constant losses

4.	In dc machines, carbon is used to make ________.

5.	_________ test is the most economical method for estimating the efficiency of a large capacity DC machine.

6.	Stepped core is used in transformers, in order to reduce the volume of ________.

7.	The all-day efficiency is calculated for _________ type transformers.

8.	Satisfying all the necessary conditions for parallel operation of transformers, sharing of load between the transformers will be based on their individual ______ ratings.

a.	kV		b.	kA		c.	 kW		d.	kVA

9.	Short circuit test of transformer is otherwise called as _______ test.

10.	_________ Transformers are used to extend the range of moving iron voltmeters.

PART – B  (5 x 3 = 15 MARKS)

11.	State the merits and demerits of Swinburne’s test.

12.	List the various fixed and variable losses in DC motors.

13.	Differentiate between shell and core type transformers.

14.	What is polarity test? Why transformers are subjected to this test?

15.	What are the various types of connections used in three phase transformers?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the constructional details of a DC generator with a neat diagram.

(OR)

17.	What is meant by armature reaction? Discuss the effects of armature reaction on the working of a DC generator.

18.	Explain the principle of operation of a DC motor with a suitable diagram. Also explain the characteristics of DC motor.

(OR)

19.	Discuss with an aid of circuit diagram the Hopkinson’s test for a pair of shunt motor and shunt generator. State the merits and demerits of the test.

20.	From the first principles, derive the emf equation and transformation ratio of a transformer.

(OR)

21.	Explain clearly how all-day efficiency is different from conventional efficiency?

[P.T.O.]

22.	A single phase 3.3 kV/240V transformer has a primary resistance of 2.25 ohm and secondary resistance of 0.012 ohm. Determine:

a.	the equivalent resistance of the primary referred to the secondary.

b.	the equivalent resistance of the secondary referred to secondary.

c.	total transformer resistance when referred to secondary.

d.	total transformer resistance when referred to the secondary.

(OR)

23.	What is Sumpner’s test? Explain its principle with the help of a circuit diagram.

24.	Explain the construction and principles of the operation of current transformers and potential transformers.

(OR)

25.	Explain the various types of 3-phase transformer connections.