Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	INDUCTION AND SYNCHRONOUS MACHINES		     Time : 3 hours

Subject Code:	EE251											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention any two types of starters used for three-phase induction motor.

2.	In a three-phase induction motor, the relative speed of stator flux with respect to ___________ is zero.

3.	How would you reverse the direction of rotation of a capacitor start-induction-run motor?

4.	Mention any two types of single-phase induction motor.

5.	Can a dc generator be converted in to an alternator?

6.	Write the emf equation of alternator.

7.	A synchronous motor is self starting – True or False?

8.	What is the use of damper winding in a synchronous motor?

9.	Define direct-axis reactance.

10.	For leading power factor load, voltage regulation of an alternator is ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw a family of slip-torque characteristics of three-phase induction motor for different values of rotor resistances.

12.	Write a note on magnetic levitation.

13.	Discuss the effects of armature reaction in alternator.

14.	How a synchronous motor be used as a synchronous condenser?

15.	Draw the phasor diagram of a salient-pole alternator under zero power factor leading.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	The short-circuit current of a small 3-phase induction motor is 3 times the full-load current. 	Determine the starting torque as a percentage of full-load torque if full-load slip is 3 %.

(8)

b.	What is an induction generator? What are its applications?				         		    (7)

(OR)

17.	Explain different speed control methods for a three-phase induction motor.

18.	Write a brief note on repulsion type induction motor.

(OR)

19.	The following test results were obtained in case of a 220 V single-phase induction motor:

No-load test			: 220 V, 310 W, 5.8 A

Blocked-rotor test	: 120 V, 530 W, 13.8 A

Determine the approximate equivalent circuit of the motor.

20.	Derive the expression for emf induced in an alternator explaining clearly all the terms present in the equation.

(OR)

[P.T.O.]

21.	Derive the expressions for synchronizing power and torque of alternators in parallel. A 3,000 kVA, 6-pole alternator runs at 1000 rpm in parallel with other machines on 3,300 V bus-bars. The	 synchronous reactance is 25 %. Calculate the synchronizing power for one mechanical degree of displacement and the corresponding synchronizing torque.

22.	Discuss the effect of excitation on armature current and power factor of a synchronous motor and hence explain the V curves and inverted V curves.

(OR)

23.	Explain the principle of operation of synchronous motor and hence develop its phasor diagram.

24.	Explain slip test to find the regulation of salient pole alternator.

(OR)

25.	A 440 V, 50 Hz, delta connected synchronous generator has a direct-axis reactance of 0.12 Ω and a quadrature-axis reactance of 0.075 Ω/phase; the armature resistance being negligible. The generator is supplying 1000 A at 0.8 lagging p.f.

a.	Find the excitation emf neglecting saliency and assuming Xs=Xd.

b.	Find the excitation emf accounting for saliency.