Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRICAL MACHINE DESIGN						      Time : 3 hours

Subject Code:	EE252											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define specific electric loading.

2.	Mention the purpose of ventilating ducts.

3.	What are the parts of a DC Machine that are involved in forming the magnetic circuit?

4.	What is the need of commutator?

5.	What is meant by window space factor?

6.	What is meant by voltage regulation of a transformer?

7.	State the main constructional differences between cage induction motor and slip ring induction motor.

8.	Define winding factor.

9.	What is meant by saline pole machine?

10.	Define Short circuit ratio.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the output equation of a DC machine.	.

12.	Define: Air gap.

13.	Write the output equation of a single phase transformer.

14.	Explain the phenomenon of cogging.

15.	The number of conductors per slot in a 3 phase induction motor is 10 and number of stator slots 	= 36. Calculate number of turns in series per phase.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Derive the relationship between real and apparent flux densities. 		     			    (7)

b. 	A 6-pole DC machine has the following design data. Armature diameter = 35 cm, armature core length = 20 cm, length of air gap at pole centre = 0.25 cm, flux per pole = 15 milliweber, field form factor = 0.66. Calculate the ampere turns required for the air gap, if the armature surface is slotted and the gap expansion factor is 1.02.					    (8)

(OR)

17.	a. 	List the choice of specific electric and magnetic loadings.    			    			    (7)

b. 	Determine the air gap length of a DC Machine from the following data: Gross length of core = 0.12 m, no. of ducts = 1, width of duct = 10 mm, slot pitch = 25 mm, slot width = 10 mm, Carter’s coefficient for slots and ducts = 0.32, gap density at pole centre = 0.7 T, field mmf per pole = 3900 AT, mmf required for iron parts of magnetic circuit = 800 AT. 																			    (8)

18.	a. 	Compare lap and wave windings. 												    (7)

b. 	Design a suitable commutator and brushes for a 400 kW, 500V, 8 pole, and 375 rpm dc shunt generator.  D = 80 cm, L = 25 cm, number of commutating segments = 192.	    (8)

(OR)

[P.T.O.]

19.	Explain the various factors that are affected by the selection of number of poles in DC machines.

20.	Derive an expression for the number of cooling tubes needed for a transformer tank.

(OR)

21.	a. 	Derive the output equation of a single phase transformer.		             				    (7)

b. 	The tank of a 125 kVA oil immersed power transformer has the  dimensions 100 x 55 x 125 cm.  Design a suitable arrangement of cooling tubes of mean length 100 cm to limit the temperature rise to 35 C if the full load loss to be dissipated is 3.75 kW.		    (8)

22.	Estimate the stator core dimensions, number of stator slots and number of stator Conductors per slot for a 100 KW, 3300 V, 50 Hz, 12 pole, star connected slip ring Induction motor, the specific magnetic loading is 0.4 Wb/m2 and specific electric Loading is 25,000 amp. cond/m. Efficiency = 0.9, powerfactor = 0.9,  Kws = 0.96. Choose main dimensions to give best power factor. The slot loading should not exceed 500 ampere conductor.

(OR)

23.	a. 	Derive the output equation for an AC machine.	   			    					    (7)

b. 	Explain the procedure to draw the circle diagram for induction motors.	    			    (8)

24.	Explain Salient pole machine in detail with relevant diagrams.

(OR)

25.	Explain the following:

a.	Turbo-alternator.		 				     										    (7)

b.   	Damper winding.					                									    (8)