Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	POWER ELECTRONICS								      Time : 3 hours

Subject Code:	EE254											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The SCR can be turned off by ______________.

2.	Name the voltage controlled power semiconductor device preferred for medium power application with highest switching frequency.

3.	What is the purpose of a freewheeling diode?

4.	The firing angle at which rectifier will work in inversion mode is _____________.

5.	The formula for average output voltage of a Buck boost converter is ____________.

6.	Name the power electronic converter which converts fixed frequency ac input voltage into variable frequency output voltage.

7.	What is the type of commutation used in series inverter?

8.	The triple harmonics are automatically cancelled in ____________ DC to AC converter  circuit.

9.	The zero crossing detector is required to design inverter circuit (True/False).

10.	Mention the general range of power rating of the UPS used in house hold.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish Transistor, SCR and GTO based on the turn off mechanism.

12.	Define ripple factor and distortion factor with necessary equations.

13.	A  DC chopper has an  input voltage  of  200V. It remains in on state for 200ms and off state for 10 ms. Determine average load voltage.

14.	What are the different methods of controlling harmonics in inverter?

15.	Write the algorithm to generate control signals for a single phase fully controlled AC to DC converter.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the construction of SCR with a neat sketch. Discuss the different modes of operation of SCR with the help of static VI characteristics.

(OR)

17.	Explain the operation of power MOSFET with its output and transfer  characteristics.

18.	a.	Explain the operation of single phase fully controlled converter with a neat diagram along 	with relevant waveforms for firing angle α =30 ˚. Also derive the expression for average 	output voltage.																(10)

b.	A single phase fully controlled converter is fed with 230V, 50 Hz supply. The load of the 	converter is purely resistive R=10 Ω . Obtain the average output voltage for firing angle

α =45ο.																		(5)

(OR)

19.	Draw the circuit diagram and explain the operation of a  three phase fully controlled converter with relevant waveforms for firing angle 30ο  and  90ο. Derive the expression for average output voltage.

[P.T.O.]

20.	Discuss the principle of operation of DC-DC step up chopper with suitable waveforms. Derive the expression for its average and rms dc output voltage.

(OR)

21.	a.	Explain time ratio control and current limit control strategies in chopper. 			  (10)

b.	A step down chopper has input voltage of 230V with 10Ω. For a duty cycle of 0.5  calculate 	i.	average and rms value of output voltage 	ii.	power delivered to the load.    																		  (5)

22.	Explain different methods of voltage control adopted in an inverter with suitable diagrams and waveforms.

(OR)

23.	Draw the circuit diagram and explain the operation of a three phase inverter in 180 ο mode of operation. Draw the phase and line voltage waveform and derive the expression for rms output voltage.

24.	Enumerate the benefits of HVDC Transmission. Explain with a neat diagram the various configurations of HVDC systems.

(OR)

25.	Explain the various configurations and operation of UPS with a neat diagram.